###### Experiment VII:

###### Diode and Rectifier Circuits

##### I.  References

Horowitz and Hill, The Art of Electronics.

Fortney, Principles of Electronics: Analog and Digital

Tipler, Physics for Scientists and Engineers, Vol. 2.

##### II.  Equipment

Digital Oscilloscope			Digital Multimeter

Signal Generator			Resistance and Capacitance Box

Differential Amplifier			10mH Inductor

Resistor/Capacitor Board		Diode board

##### III.  Introduction

The circuit elements we have studied so far (resistors, capacitors, and inductors) are all linear, meaning that a doubling of the applied signal (a voltage, say) produces a doubling of the response (a current, say). This is true even for the reactive devices (capacitors and inductors). These devices are also passive, meaning that they don't have a built-in source of power and they are all two-terminal devices.  As we found in experiment I, the I-V curve of a diode is highly non-linear.  The diode (Fig. VII-1a) is a very important and useful two-terminal passive non-linear device.  The diode's arrow (the anode terminal) points in the direction of forward current flow: from the P-type material to the N-type.  An N-type material is one where negative charge carriers such as electrons are present and a P-type material is one where positive charge carriers are dominant.  The characteristic current-voltage (IV) curve of an idealized diode is shown in Figure VII-1b. This idealized device will conduct current without impedance in the forward direction and allows no current in the reverse direction.  For many circuit applications, this ideal diode model is an adequate representation of an actual diode and simply requires that the circuit analysis be separated into two parts: forward current and reverse current.  For other applications, a more accurate description may be needed.

The ideal P-N junction has an I-V relationship (shown in Fig VII-1c) that can be given by

I=I0(eVVT−1),						 	(VII-1)

where VT is a threshold voltage and both I0 and VT are temperature dependent.  Nominal values for a Si diode at room temperature are I0= 1 nA and VT=50 mV.  The operation of an actual diode can be summarized by the I-V curve shown in Fig. VII-1d.  This curve is characterized by two linear regions of constant slope connected by an exponential curve in the region of small forward bias.  The PN diode is thus our first example of a nonlinear circuit element: the current through the device is not always proportional to the voltage across the device.  Some circuits make use of the nonlinear region of this curve, but for most applications this region of the curve can be avoided and the device can be modeled as a “piece-wise” linear circuit element.

### Figure VII-1:  Properties of ideal and real diodes.

Noting that the slope at any point on this IV curve has the units of inverse resistance, we can assign a "dynamic" resistance to the diode in each of its linear regions: Rf in the forward-biased region and Rr in the reverse-biased region. These resistances are defined by Rf=ΔVΔI and Rr=ΔVΔI in their respective regions as shown in Fig. VII-1d.  Typical signal diodes have a forward resistance of a few ohms and a reverse resistance around a megaohm.

The equivalent circuit model of an actual diode forward-biased into the linear region is shown in Figure VII-2a.  This model reproduces the linear portion of the IV curve of Figure VII-1d but extends to intersect the V axis at VPN. The voltage, VPN, represents the effective voltage drop across a forward-biased PN junction (the turn on voltage) and is represented in the model by a constant voltage source. For a germanium diode, VPN is approximately 0.3 V; for a silicon diode it is close to 0.6 V. The reverse-bias equivalent circuit of Figure VII-2b is somewhat simpler and does not require a voltage source, since the linear portion of the actual diode curve passes very close to the origin.

### Figure VII-2:  Model for the linear portion of a diode; (a) Forward biased, (b) reversed biased, (c) the combined model in terms of VPN, the PN junction voltage drop.

Using two ideal diodes to restrict the current to the appropriate direction through each branch, these two linear element models can be combined as shown in Figure VII-2c.  Frequently the forward voltage drop of about 0.5 to 0.8 volts is of little concern, and the diode can be treated as a good approximation to an ideal one-way conductor. There are other important characteristics that distinguish the thousands of diode types available, e.g., maximum forward current, capacitance, leakage current, and reverse recovery time (see Table VII-1 for characteristics of some typical diodes).  The most important diode feature not included in this model is the switching time between forward current If and reverse current Ir. Actual diodes require a time ranging from a picosecond to a microsecond to switch in this direction, even when the applied voltage across the diode is assumed to switch instantly.  This represents a significant high-frequency limitation for many diodes.

III.A  Rectification

A rectifier changes ac to dc; this is one of the simplest and most important applications of diodes (diodes are sometimes called rectifiers).  The simplest circuit is shown in Figure VII-3a.  The "ac" symbol represents a source of ac voltage; in electronic circuits it is usually provided by a transformer, powered from the ac power line.  For a sine-wave input that is much larger than the forward drop (about 0.6V for silicon diodes, the usual type), the output will look like that in Fig. VII-3b.  If you think of the diode as a one-way conductor you won't have any trouble understanding how the circuit works. This circuit is called a halfwave rectifier, because only half of the input waveform is delivered to the load resistor.

(a)

(b)

### Figure VII-3:  A Half-Wave Rectifier.

Figure VII-4a shows another rectifier circuit, a full-wave bridge.  Figure VII-4b shows the voltage across the load resistor Rload for which the whole input waveform is used. The gaps at zero voltage occur because of the diodes' forward voltage drop.  The rectified waveforms shown in Fig VII-4b are not good for much as they stand.  They are dc only in the sense that they do not change polarity.  But they still have a lot of "ripple" (periodic variations in voltage about the steady value) that has to be smoothed out in order to generate genuine dc.  This we do by tacking on a low-pass filter (Fig. VII-5).

Actually, the series resistor R in figure VII-5 is unnecessary and is always omitted (although you sometimes see a very small resistor used to limit the peak rectifier current). The reason is that the diodes prevent flow of current back out of the capacitors, which are really serving more as energy-storage devices than as part of a classic low-pass filter. The energy stored in a capacitor is U = CV2.  For C in farads and V in volts, U comes out in joules (watt-seconds).  The capacitor value is chosen so that Rload C>>1/f .  This makes the time constant for discharge much longer than the time between recharging and the average voltage across the capacitor approaches a constant.

(b)

(a)

### Figure VII-4:  Full-wave bridge rectifier.

### Figure VII-5:  Circuit for building a dc power supply.

##### III.B  The Signal Rectifier

There are other occasions when you use diode to make a waveform of one polarity only. If the input waveform is not a sine wave, you usually don't think of it as rectification in the sense of a power supply.  For instance, you might want a train of pulses corresponding to the rising edge of square wave.  The easiest way is to rectify the differentiated wave (Fig. VII-6). Always keep in mind the 0.6 volt (approximately) forward drop of the diode. This circuit, for instance, gives no output for square waves smaller than 0.6 Volts peak-to-peak.

### Figure VII-6:  Square-wave signal rectifier.

In this circuit, if the frequency is high enough, the combination of R1 and C provide the derivative of the input signal.  The diode rectifies the pulse train providing only positive pulses to the load resistor R2.

##### III.C  The Frequency to Voltage Converter

An interesting application of diodes is in the fabrication of a simple frequency-to-voltage converter.  One approach is shown in figure 7.7 for in input square wave signal.  After each charging impulse from the input square wave, C2 will discharge across R for a time equal to the period of the input signal.  During this part of the cycle the dwindling charge on C2 is given by

q=q2e−tRC2,							(VII-2)

where q2 is the initial charge on C2, and t is measured from the charging edge of the square wave.  The stored charge lost into R in a time 1/f is given by the initial charge minus the stored charge after a time 1/f and is expressed by

Δq2=q2(1−e−1RC2f).					(VII-3)

If RC2f>>1, using a Taylor series expansion of Eq. VII-3, we can approximate it by

Δq2=q2fRC2 .    						(VII-4)

As you can show, the charged gained in one cycle is simply

Δq=C1C2C1+C2(V−V2).	  				(VII-5)

### Figure VII-7:  A frequency to voltage converter.

When this is set equal to the charge lost per cycle (Eq. VII-4) the resulting expression can be solved for V2 to give

V2=C1C2RfVC1+C2+C1C2Rf  .   					(VII-6)

With the further requirement that C1C2Rf << C1+C2, a linear relationship between voltage and frequency can be obtained:

V2=C1C2C1+C2RVf.   					(VII-7)

It turns out that in order for this circuit to work properly, C1<<C2 and the design requirements can be simplified to

1C2<<Rf<<1C1  						(VII-8)

Therefore over a restricted range of frequencies the output voltage of this circuit will be proportional to the frequency of the input signal.  Although this was derived for a square wave input signal, a similar expression could be obtained for a sinusoidal signal.

##### III.D  Inductive Surge Suppression

What happens if you open a switch that is providing current to an inductor? Because inductors have the property V=L dI/dt, it is not possible to turn off the current suddenly, since that would imply an infinite voltage across the inductor's terminals.  What happens instead is that the voltage across the inductor suddenly rises and keeps rising until it forces current flow.  Electronic devices controlling inductive loads can be easily damaged, especially the component that "breaks down” in order to satisfy the inductor's craving for continuity of current.  Consider the circuit in Figure VII-8a.  The switch is initially closed, and current is flowing through the inductor.  When the switch opened, the inductor "tries" to keep current flowing from A to B, as it had been.  That means that terminal B goes positive relative to terminal A.   In a case like this it may go 1000 volts positive before the switch contact "blows over".  This shortens the life of the switch and also generates impulsive interference that may affect other circuits nearby.  If the switch happens to be a transistor, it would be an understatement to say that its life is shortened; its life is ended!

The best solution is to put a diode across the inductor, as in figure VII-8b.  When the switch is on, the diode is back-biased (from the dc drop across the inductor winding resistance).  At turn-off the diode goes into conduction, putting the switch terminal a diode drop above the positive supply voltage.  The diode must be able to handle the initial diode current, which equals the steady current that had been flowing through the inductor; something like a 1N4004 is fine for nearly all cases.  The only disadvantage of this protection circuit is that it lengthens the decay of current through the inductor, since the rate of change of inductor current is proportional to-the voltage across it.

(a)

(b)

### Figure VII-8:  (a) An inductive kick.  (b)  Blocking and inductive kick.

##### IV.  Measurements with a sine wave input

IV.A:  Set up the circuit shown in Fig. VII-3a using a 1 k load resistor and a 1000 Hz sine wave input from the signal generator.

Using the measure function on the oscilloscope, measure both the rms voltage out of the generator and the rms voltage across the load resistor as a function of the generator voltage.  Use voltages that vary from 0.1 V to the maximum available from the generator and make sure that the dc offset is close to zero.  From a graph of Vload vs Vgen determine the minimum generator voltage (and your error) for which the voltage across the load is linearly proportional to Vgen.  Does this graph behave as you would expect?

IV.B:  Set up the full wave bridge circuit shown in Fig. VII-4a using a 1 k load resistor and a 1000 Hz sine wave input from the signal generator.  Connect the oscilloscope to the generator output and measure Vload using the differential preamp.  Using the measure functions on the oscilloscope, measure the peak-peak, mean, and rms of both the input voltage and the voltage across the load.

IV.C:    Remove the load resistor and place a 0.1 F capacitor across the output of the full wave bridge.  Connect the differential preamp across the capacitor and measure the peak-peak, mean and rms voltage across the capacitor as a function of frequency from 20 Hz to 60 kHz taking about 10 points.  For this measurement, attempt to keep the generator rms voltage constant at all frequencies.  Make a plot of the mean and rms voltage across the capacitor.  Comment in your report on what you observe in this plot in terms of the quality of your ac to dc voltage converter.

##### V.  Measurements with a square wave input

V.A:  Assemble the circuit shown in Fig. VII-6 using a 100 pF capacitor and two 1 k resistors.  Drive the circuit with a square wave of 20 volts peak to peak.  Measure the voltage across R1 and R2 as a function of frequency from about 200 Hz to 200 kHz.  Determine the approximate frequency range where VR1 is nearly the derivative of the input signal.  Does this agree with your expectation based upon the values of the circuit components?  Compare the peak voltage VR1 with VR2.  Is the difference accounted for by the forward voltage drop of the diode?  How does the quality of your pulses compare to the TTL pulses out of the signal generator?  (Hint:  Compare the leading edge of both signals.

V.B:  Setup the frequency to voltage converter circuit shown in the top of Fig. VII-7 with C1=100 pF, C2=4F, and R=100 k.  Using equation VII-8 we can estimate the frequency range of nearly linear behavior to be between 25 Hz and 20kHz.  Measure the actual values of these components.  Adjust the output of the square wave generator to be about 16-20 volts peak-to-peak and measure both the average voltage V2 and V, the 0-to-peak voltage out of the generator (the diode will remove the negative part of the square wave), as a function of f over the range of 10 Hz to 100 kHz taking about 5 points per decade.  Use the cursor function on the oscilloscope for this measurement ignoring the transient spikes on the scope.  In order to get the highest possible precision for this measurement, adjust the gain of the scope to be as high as possible and still keep the steady state signal on the scope.  (Hint: use the Y-offset to keep the signal on the screen at the highest gain.) Plot V2/V vs. f and determine over what frequency range V2 is approximately linear in f.  Estimate how precisely and accurately you can determine f from a measurement of V2 if you operate in the regime where your data is approximately linear.  Compare your fitted slope in the linear regime to Eq. VII-7 and comment on the differences.

V.C:   Construct a series L-R circuit driven by a square wave.  Use L=10 mH, R=1 kΩ, and set the voltage out of the generator to 1V.  Measure the voltage across the inductor and observe the “inductive kick” occurring at both the rising and falling edge of the square wave.  Vary the frequency to determine if the magnitude of the spikes change and set the frequency to the point where the inductive kick is at its maximum.  Insert a diode in parallel with the inductor and measure the magnitude of both spikes.  How much does the diode suppress the inductive kick on both the rising and falling edge of the drive signal?  Try this again after placing a second diode across the inductor with the diodes arrow (anode) pointing in the opposite direction from the first diode.  Are there any changes?  This type of circuit is commonly used to protect very high field solenoid magnets from self-destructing when the current through the magnet is suddenly changed (like due to a power failure).

### Table VII-1  Properties of a few representative diodes.