###### Experiment VI:

###### The LRC Circuit and Resonance

##### I.  References

Halliday, Resnick and Krane, Physics, Vol. 2, 4th Ed., Chapters 38,39

Purcell, Electricity and Magnetism, Chapter 7,8

##### II.  Equipment

Digital Oscillocope			Digital Multimeter	Variable Resistor

Signal Generator			0.01μF Capacitor

Differential Amplifier			10mH Inductor

Resistor/Capacitor Board		Variable Inductor

##### III.  Introduction

Figure VI-1 shows a circuit with driving voltage V(t)=V0cos(ωt) connected in series with a resistor, inductor, and capacitor.  In this circuit, the resistance R includes every known resistance in the circuit, which can come from the internal resistance of the generator, the resistance of the inductor, and any real resistors connected.

### Figure VI-1:  LRC circuit driven by a sine wave input.

As before, we analyze the current in the circuit by using Kirchoff’s laws for complex resistances.  The equivalent complex resistance is given by the series addition of R, XL = L, and XC=1/C:

Req=R+iXL−iXC=R+i(XL−XC)=Zeiθ

where

Z=R2+(XL−XC)2						(VI-1)

and

tanθ=(XL−XC)R							(VI-2)

We can then find the current in the circuit using V(t)=I(t)Req:

I(ω,t)=V0eiωtZ(ω)eiθ(ω)=V0Zei(ωt−θ).						(VI-3)

The current will therefore have an amplitude I(ω), which is a function of the frequency:

I(ω)=V0Z(ω)=V0R2+(XL−XC)2						(VI-4)

and which has its maximum at the resonance condition:

IMAX=V0R							(VI-5)

which is also when XC = XL.  The resonance frequency  =0 is

ω0=1LC							(VI-6)

Figure VI-2 shows the plot of the phase angle  vs. /0 .  Note that when the frequency is very low, XL 0 and XC  , so tan  and  /2.  At Very high frequencies, XC  0 and XL  , so tan  +, and   +π/2.  At resonance, XL = XC,  =0,

tan  0, and   0 (no phase shift).

### Figure VI-2:  Behavior of the phase shift as a function of frequency.

Another feature is that at very low (high) frequencies, either XC (XL) gets very large, which increases the impedance Z, causing the amplitude of the current to become very small.  Somewhere between low and high frequency is a resonance frequency 0, at XC = XL. The plot of  vs.  will therefore have a maximum at the “resonance frequency” 0, as shown in figure IV-3.

### Figure VI-3: Behavior of the current as a function of frequency, showing a resonance peak.

The width of the resonance, , is defined as the difference between the two angular frequencies  and , which are defined as those frequencies at which the amplitude of the current is IMAX/2.  These two angular frequencies can be calculated using equations VI-4 and VI-5:

V02R=V0R2+(XL−XC)2

or equivalently:

1+(XL−XC)2R2=2

which translates to

(XL−XC)2R2=1.

One has to be careful taking the square root of both sides.  If we define α=±1, then

ΧL=ΧC+αR.

Using the equations XL  L,  XC   1/ C, defining L = L/R , and ω0≡1/LC we get the quadratic equation

ω2−ωατL−ω02=0

which has the solutions

ω=α2τL±ω02+1(2τL)2.

The negative sign here is unphysical since it produces negative frequencies, so we only take the positive solution.  But the coefficient a can be either +1 or –1, which gives us the two solutions :

ω±=±12τL+ω02+1(2τL)2.

The width  is the difference between the two:

Δω=ω+−ω−=1τL.						(VI-7)

So the width of the resonance depends only upon L and R.  However, remember that the resonant frequency 0 also depends on L (and C).  So the resonant frequency and the width of the resonance are linked by the value of the inductance used in the circuit.  A more useful quantity is the “Quality factor”

Q≡ω0Δω.							(VI-8)

This quantity is essentially the ability of the circuit to limit the flow of high current to a small band of frequencies about the resonance frequency 0.  As Q→0, the current flowing through the circuit will never be large and a large range of frequencies will behave in the same way.  As Q  , the circuit will resonate only for frequencies that are near the resonance frequency and the current through the circuit can become large.  This type of resonant behavior is found to be extremely useful for things like radio reception, where a receiver LCR circuit can be tuned to a particular radio transmission frequency, without interference from frequencies nearby.

Combining the above equations gives:

Q≡ω0Δω=ω0τL=LRLC=LR2C.					(VI-9)

Using this definition of Q we can rewrite equation VI-1 as:

Z=R1+Q2ωω0−ω0ω2.

This equation shows explicitly that for a large Q, the current will be very small (“off resonance”, Z→RQ, therefore Z will be very large) except at the resonance frequency  =0 (where Z=R).

Knowing the current I(t) we can calculate the voltage drops across the various components.  The voltage drop across any resistor RR (we use this notation to distinguish between a real resistive element RR and a total resistance R):

VR(t)=I(t)RR=V0RRZcos(ω t−φ).				           (VI-10)

We see from this equation that VR(t) is out of phase with respect to the driving (input) voltage V(t) by an amount  which can be either positive or negative according to whether the angular frequency is less than or greater than the resonance angular frequency 0 as seen by equation VI-2.  At resonance, Z=R (total resistance in the circuit) and =0, which gives

VR(t)|ω=ω0=V0RRRcosω t ,

in phase with the input signal and reduced by an amount which depends upon the “other” resistive elements in the circuit.

Turning to the capacitor C and inductor L, the voltage drops are:

VC(t)=I(t)−iΧC=V0ΧCZe−iπ/2ei(ωt−φ)V0ΧCZcos(ωt−φ−π/2)

=+V0ΧCZsin(ωt−φ)				           (VI-11)

and

VL(t)=I(t)iΧL=V0ΧLZeiπ/2ei(ωt−φ)V0ΧLZcos(ωt−φ+π/2)

=−V0ΧLZsin(ωt−φ)				         (VI-12)

Using XL ωL, XC1/ C,τL≡L/R, and ω0≡1/LCwe can write

ΧLZ=ωLZ=ωω0ω0LRRZ=ωω0ω0τLRZ=Qωω0RZ

and

ΧCZ=1ZωC=ω0ωLω0LCRRZ=ω0ω1ω0LCLRRZ=ωω0ω0τLRZ=Qω0ωRZ

to get the following set of equations for the voltage drop across the resistor RR, the inductor L, and the capacitor C:

VR(t)=V0RRZcos(ωt−φ)VL(t)=−QV0ωω0RRZcos(ωt−φ)VC(t)=+QV0ωω0RRZsin(ωt−φ)

Figure VI-4 shows the voltages as a function of t.   The input voltage VIN (dot dashed line) starts at V0 when t=0.  The voltage drop across the resistor VR is offset by the phase angle= 0.5.  Since in this case  is positive, VR lags VIN.  The voltage across the capacitor VC and inductor VL are shown as dashed and solid lines respectively.  Note that these are completely out of phase with each other, and both are out of phase with respect to VR (by an amount ±π/2).

### Figure VI-4:  Behavior of the voltages across the various circuit elements.

Notice that the amplitudes for VC and VL are both greater than that for VIN.  To understand what is going on, consider what happens at resonance where XL=XC, =0, and  = 0.  The equations for the voltage drops are

VR(t)|ω=ω0=V0RRRcosω0t

VL(t)|ω=ω0=−QV0RRRsinω0t

VC(t)|ω=ω0=+QV0RRRsinω0t.

Written this way, we can clearly see that at resonance, the amplitude for VL and VC are proportional to the driving amplitude V0 and Q, which is “tuned” to be as large as possible. Figure VI-5 shows the corresponding curves.  Notice that VR is in phase with VIN, but with a slightly lower amplitude.  This is because VR is proportional to the voltage drop across the resistor RR, whereas the impedance Z takes into account the total real resistance (R) in the circuit.  For these cases, RR is 80% of the total resistance R.

### Figure VI-5: Voltages when the circuit is resonant

The large amplitudes for VL and VC can be understood by turning to classical mechanics.  Imaging a mass m on a spring with spring constant k, driven by an external driving force F(t) at some angular frequency .  The equation of motion is

:

F(t)=mx+kx

where x measures the displacement from equilibrium. The system will oscillate with greatest amplitude when =0 with 0, the natural frequency given by 0=km.  Now, add a piece of cardboard attached to the mass, with a large area such that wind resistance comes into play.  The wind resistance force will be proportional to the velocity, which modifies the equation of motion to be:

F(t)=mx+kx+bx

The wind resistance is a “damping” term bx: all energy will be lost only through this term, not from the “inertial” term mxor the “elastic” term kx.  In words, the equation of motion is then:

Driving Force (F(t))= inertial term (mx) + elastic force (kx) + damping force (bx)

Now turn back to the RLC circuit, and use Kirchoff’s law with regards to voltages around the loop and write the differential equation:

VIN(t)=VL+VC+VRLI+qC+IRLq+1Cq+Rq

We can then equate VL with the inertial term (inertia involved in building up the magnetic field inside the inductor), VC with the “elastic” term (the elasticity involved in moving an equal and opposite charge on and off the plates of a capacitor) and VR with the “damping” term (the resistor dissipates energy whether the current goes in one end and out the other or vice versa).  All energy is dissipated through the resistor.  Note that in any mechanically oscillating system, it is quite natural for the resonance oscillations to be larger in amplitude than the driving force: energy is being added to the system little by little, dissipation is minimal, and the energy causes the total oscillation amplitude to build up.  The classic example of this mechanical system is a person on a swing, being pushed.  The pushing (F(t)) amplitude need not be large compared to the oscillation, the system builds up a lot of energy at resonance.

##### IV.  Measurements with a sine wave input

Part A:  Initial Setup

A-1:  Set up a circuit with an inductor, resistor and capacitor in the loop.  As input, use a 2 kHz sine wave with a maximum amplitude of about 1.5 V.  If necessary, eliminate any DC offset of the input voltage by using the DC offset adjust.  For components, start with a 1 k resistor, a 0.01 F capacitor, and the 200 mH inductor.

For all measurements, be sure that all inputs are DC coupled, and that the oscilloscope is triggered externally by the TTL output of the waveform generator.

A-2:  Measure the values of L and C using the LCR meter.  Measure the value of the resistor RR and the resistance of the inductor (RL) using the digital multimeter.  Determine the total resistance of the circuit, R = RR + RL + R0, where R0 is the internal resistance of the waveform generator (which is 50).  From these measured values, determine the resonance (angular) frequency 0 and the gain Q.

Part B:  Resonant frequency and Q value from voltages

B-1:  Look at the input voltage V0 and the voltage across the resistor, VR , on the oscilloscope.  Be sure to pay attention to where “ground” is located in your circuit and use the instrumentation amplifier if necessary.  Determine the resonant frequency f0 = 0/2 by looking for the frequency at which the phase shift  goes to 0.  Estimate the uncertainty in f0 by from your estimate of how well you can determine the zero crossing.

B-2:  Measure the amplitude of VR at the resonant frequency, and then find the two frequencies f at which the amplitude drops to 12 of its maximum value.  Record these two frequencies, and compute the width  of the resonance.  From the width and the determination of 0, determine the Q value and its uncertainty.  Compare this value to what you got in part A, and comment on any discrepancy.

Part C:  Amplitude and phase vs. frequency

C-1:  Vary the input frequency using the following values:

f = f0  (0.1, 0.3, 0.5, 0.7, 0.9, 0.95, 1.0, 1.05, 1.1, 1.3, 1.5, 1.7, 1.9, 2.0, 2.3)

For each value, record the amplitudes of V0, VR, VC, and VL , as well as the frequency f and the phase shift  (from the time shift of the zero crossings) between V0 and VR.  From these values, compute the impedances XL and XC.

C-2:  As can be seen in equation VI-2, note that there is a linear relationship between tan and XLXC, for which the slope is 1/R (where again R is the sum of all resistance in the circuit).  Make a graph of tan  vs XLXC, fit it to a line, and from the slope, determine R and its uncertainty.  Compare the value of R to what you computed in part A above.

C-3:  Plot the phase shift  vs. .  From the zero crossing of this graph, determine the resonant frequency 0.  Qualitatively compare the value from your graph to what you computed in part A above.

C-4:  Plot the amplitude of VR(t) vs.  compare From the graph, determine the resonance frequency compare 0 and the width of the resonance , and from these determine the Q value.  Compare to the values determined in parts A and B and comment on any discrepancies.

##### V.  The LCR Circuit with a Square Wave

If you hit a bell with a hammer, it will ring.  The ringing will die down over time because energy is lost through friction (air resistance, internal friction of the bell ringing, etc.)  The ringing will be clearly heard as a “note”, at some frequency, and will die down over time.  We therefore predict that the amplitude of the bell’s oscillation will be a sine wave with an amplitude that decreases exponentially in time.  The general solution of the equation will be

x(t)=x0+x1e−t/tsinwosct.

As we have already discussed, the LCR circuit shown below is completely analogous to the oscillating mechanical system.

### Figure VI-6:  LRC circuit to be used in this lab

Using Kirchoff’s laws for summing voltage changes around a loop results in

VIN(t)=VL+VC+VRLI+QC+IR

LQ+1CQ+RQ .			 	          (VI-13)

The LCR analog of hitting the bell with a hammer would be some kind of source that suddenly “turns on”, changing its DC level like a step function:

### Figure VI-7:  Square wave input used as the impulse to cause the circuit to ring.

where the voltage from the source rises to some constant V0 at t=0.  Since the LCR analog to the mechanical oscillator’s amplitude x(t) is Q(t), the charge on the capacitor, we would guess the following for Q(t):

Q(t)=Q0+Q1e−t/τsinωosct

where Q0 and Q1 are determined by the initial conditions (t=0), andt and wosc are determined by the dynamics (L, R, and C).

Differentiating once gives:

Q=Q1e−t/τ−1τsinωosct+ωosccosωosct

and again gives

Q=Q1e−t/τ1τ2−ωosc2sinωosct−2ωoscτcosωosct

Setting VIN=V0 for t>0, and plugging all of the above 3 equations into equation VI-13, we get the following:

V0=1CQ0+Q1e−t/τL1τ2−ωosc2−R1τ+1Csinωosct+Rωosc−2Lωoscτcosωosct

This equation can only be true for all times if the following equations are true:

Q0=CV0					           (VI-14)

Rωosc−2Lωoscτ=0				           (VI-15)

L1τ2−ωosc2−R1τ+1C=0				           (VI-16)

From equation VI-15, we can solve for t to get:

τ=2LR,					           (VI-17)

and from equation VI-16 we can solve for wosc to get:

ωosc2=ω02−1τ2				           (VI-18)

where ω0=1/LCis the resonance frequency.   We see from equation VI-18 that the free oscillation of this circuit (the “ringing”) is near the resonance frequency, but reduced by an amount which is proportional to the resistance R.  This makes sense: the resistance is the only mechanism for energy loss, and ringing is associated with an exchange of energy in the system.

Figure VI-8 shows a square wave VIN driving input where the waveform changes from 0 to V0 in a period Tgen=1/fgen.  The circuit rings every time there is a transition.  The charge oscillates with period Tosc=2πωoscwith an exponentially decreasing amplitude with decay time t.  Note that 1/τ=R/2L, which means that the larger the resistance (damping), the smaller the amplitude of the ringing: for a large enough R, the circuit will be overdamped with little or no ringing.

2πωosc

### Figure VI-8:  Oscillations due to ringing of the LRC circuit.

One can calculate the resistance needed by considering that an overdamped circuit will decay away exponentially before the oscillation has a chance to go through a half wavelength, or in other words, when t=12Tosc, we want the exponential term to be “small”.  If we use e1 as “small”, then our condition is given by

(Tosc2)τ=1

Using equations VI-17 and VI-18 gives us the condition:

R2=4LC1π2+1

This means that for R less than the above equality, the circuit will be underdamped, and will ring, and for R greater than the above equality, the circuit will be overdamped, and not ring much at all.

##### VI.  Measurements with a square wave input

Set up a circuit with a square wave generator driving a serial LCR circuit.  Use a 200 Hz square wave output from the frequency generator.  Set the amplitude to be a few volts, but make sure that the maximum voltage of the oscillating part of the circuit does not exceed 10 V.  Externally trigger the scope using the frequency generator’s TTL output, and be sure that all inputs are DC coupled.  Use the variable resistor box, and set the resistance to about 200 ( this is RR).  Use and inductance of about 100-200 mH and a 0.01μF capacitor.

Part A:  Signal Characteristics from the components

Measure all three values using the meters, and measure the resistance of the inductor (RL).  Compute the total resistance in the circuit (R), which consists of the RL+RR+RO where RO is the internal resistance of the frequency generator, which is 50Ω. From these measurements, calculate woscand t and their uncertainties.

Part B:  Scope Measurements

B-1:  Display the voltage VC on the scope.  Be sure to pay attention to the grounds, as always, and use the instrumentation amplifier if needed.  Adjust the components in your circuit until you get a scope display that looks like Figure VI-8 (if you’ve made adjustments, be sure to repeat part A with the components that you actually use).  Once you get a nice “ringing”, use the scope “CURSOR” controls, and for each value of the peak Vi in the ringing, record |Vi| and the time ti.  You should be able to record about 10 of these values.  Check to be sure that none of the amplitudes of the oscillating portion of the circuit is too close to 10 V or you might be saturating the instrumentation amplifier.

B-2:  Extract t and its uncertainty from your measurements by, plotting ln|Vi| vs ti and performing a least squares fit.  The slope should be -1/τ.  Compare your result to the value you calculated from part A above.

B-3:  Extract wosc  by determining the oscillation period Tosc (along with an uncertainty) from the various values of ti.  It is your choice as to how best to do this.  Compare the value for wosc to what you calculated in part A.  As always, comment on the significance of the discrepancy.

Part C:

Once you have the circuit ringing, play around a bit with the input components (R, L, C).  Also vary the input frequency, the amplitude V0, and the DC offset.  Comment in your lab report as to how the ringing changes as you change the various components.  Find a set of values for (R,L C) that will make the circuit critically damped and overdamped.