# The Hall Effect in Metals

## References

C. Kittel: Introduction to Solid State Physics, pp. 148-151.

Ashcroft and Mermin: Solid state Physics, pp. 6-15.

Dekker: Solid State Physics, pp. 301-302.

Yarwood: High Vacuum Technique, pp. 1-38, 77-78; 81-83; 96-100; 150-158.

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- Answer Question A in section VII. (In the first part of the question, express the answer as an algebraic relation between the quantity to be determined and the measured quantities.)  Be sure that all numerical answers are labelled with the appropriate units.

- Assuming in the figure below that I = Ix and B = Bz, show how a measurement of the “sign” of the Hall voltage, H, gives the sign of the charge carriers.

- Show how you could extract a value of RH (or n) from plots of   vs. Bz and   vs. Ix; compare the accuracy of this method of obtaining RH with that resulting from simply substituting values into the equation obtained in answer to Question A.

Figure X-1

## Purpose

To use the Hall effect to show that an electric current passing through metallic silver is carried by electrons, and to determine the number density and mean drift velocity of these electrons.

## Introduction

The conductivity of a metal such as copper can be explained on the basis of a free electron gas within the metal if these electrons are allowed to have collision with the atoms of the metal lattice.  In this experiment we determine that electrons are the charge carriers in metals.  By measuring the sign of the charge carriers, their mean drift velocity and their number density can be determined based on the assumption that each carries a single electron charge.  From the negative sign of the charge carrier and their number density, it is difficult to conceive of anything other than electrons as the charge carriers.  This is consistent with other known experimental results.

## Principles

### The Hall Effect

When an electric charge q moves with velocity v in a magnetic field B the charge experiences a force perpendicular to its direction of motion such that

F=qv×B				X-1

Thus, in a slab of metal carrying an electric current Ix (Fig. X-1) the current density is jx and if the current carriers have charge q

jx=nqvx				X-2

where n is the number density of charge carriers and <vx> is their mean drift velocity in the x-direction.  If a magnetic field Bz is applied in the z-direction some of the charge carriers will be deviated towards the shaded side of the slab (Fig. X-2)

Figure X-2

where the surface charge density will build up until the electric field repelling further charges from that surface is sufficient to counteract the force of the magnetic field, then

qEH=qvxBZ			X-3

The electric field developed by the surface charge density is known as the Hall field EH, and the potential H  across the slab is the Hall voltage

φH=EHd				X-4

Since for negative charge carriers, i.e., q negative, <vx> is negative and therefore the shaded surface of the slab becomes negative while for positive charge carriers <vx> is positive and the shaded surface becomes positive, the sign of the charge carriers can readily be determined.  Further the magnitude of <vx> can be determined from

vx=EHBZ				X-5

Then if the charge q is taken as one electron charge e, the density of charge carriers can be determined from Eq. X-2.

For a given material, i.e., fixed n and q, the Hall voltage H depends only on the magnetic field Bz and the current density jx.  Therefore to make H a measurable voltage the current density must be made large.  To achieve this high current density without the need for large sample currents (Ix = jx d t), the thickness t of the slab of metal is made very small (~1000 Å) by using a thin film of the metal.

### The Production of Thin Films

Thin films of thickness approximately 1000 Å are extremely fragile and although they can be made free of a substrate, usually in the form of a skin stretched on a supporting frame, they are very difficult to handle in that form.  For this experiment it is advantageous to have the metal film closely attached to a substantial non-conducting substrate, partly because this allows several independent electrical connections to be made to the film and partly as a means of removing any heat developed in the metal film by ohmic heating.  At the same time a very pure, clean and uniform metal film can be made by evaporating the metal under vacuum onto a clean glass substrate (microscope slide).  See the instructor or TA for explicit instructions on how to evaporate the metal film, and how to measure the thickness of the sample.

## Experimental Procedure

The preparation and production of the thin silver film used in this experiment, the setting-up of the measuring system, the recording of data, and the measurement of the film thickness after a second evaporation can all be completed in a four-hour laboratory period providing one is adequately prepared.  There is no convenient point to divide the experiment to spread it out over two laboratory periods due to the rapid oxidation of the silver films.

Figure X-3

It is suggested that one period be spent producing a test silver film by evaporation.  Using this test film, practice loading this sample into the Hall-effect sample holder and taking measurements as discussed below.  Experience has indicated that first runs are often unsuccessful.  In the second lab period make a second evaporation of the silver and proceed with the full set of measurements.

### Making the Measurements

A small electromagnet (E.S.I. Model #4 HF) provides the Bz field.  It has 4-inch pole pieces with a 0.69 inch gap and produces 4000 gauss with the maximum permissible current of 15 amps.  A Hall-probe gauss meter is used to measure its output.  Be careful not to wear a wristwatch or to place steel objects near the magnet: 4000 gauss is a surprisingly strong magnetic field.

Connect the circuit shown in Fig. X-2 and insert the Hall-effect sample holder between the magnet pole pieces.  Using current of no more than 100 mA plot at least two curves for H against Ix using different Bz, and at least two curves for H against Bz using different Ix.  Sufficient data (typically around 10) should be taken to define the curves.  Also, determine the resistivity  of your silver sample, and compare this with the accepted bulk value given for Ag in some of the references.  Attempt to explain any discrepencies between your value and the accepted value.  This may be done by measuring the potential drop φρ along a measured length of the silver sample using the potentiometer.  From your data, and the measured size of the sample, determine the sign of the charge carriers and their number density assuming each is slightly charged.  Use figures X-2 and X-3 to compute the Hall coefficient for silver, RH, where RH is defined from the relation

EH=RHjx×Bz,			X-6

and compare with the theoretically accepted value.  Many spurious effects can cause signals which are large compared to the true Hall voltage.  The contacts used to measure the Hall voltage may not be exactly opposite each other, so what is measured is actually the true Hall voltage plus part of the voltage drop along the film due to Ix.  Also the current Ix may not be uniform over the film, or exactly parallel to the length of the film.  Be absolutely sure you are measuring the real effect, which is dependent on both Ix and Bz.

When you have successfully finished all the required measurements on your Hall effect sample, proceed with the measurement of the film thickness t.  Until our 405 laboratory film thickness device is repaired, you will need to make an appointment with the Instructor to use the Alpha-Step film thickness measurement apparatus in room 0205.  This measurement is necessary before RH and  can be calculated, but it is best left alone until this time in case the Hall effect sample is damaged during the setting-up procedure, in which case a second Hall effect sample and a new monitor will have to be made.

## Discussion Questions (to be answered in lab report)

A.  Measurements made in this experiment are

- Hall voltage H

- Magnetic field Bz

- Voltage drop across the standard resistor Rs, S

- Voltage drop along the silver sample 

- The dimensions of the sample, d, t, and ℓ.

How can these measurements be used to determine the number density of charge carriers, n, the mean drift velocity <vx>, and the resistivity of silver ?

If	Bz	=	2000 gauss

Rs	=	1 and S = 100 mV

H 	=	20 x 10-6 volts

	=	0.1 volts

d	=	1 cm; 	ℓ= 2 cm; 	t = 1000 Å

what are , n, <vx> and RH?

B.  Briefly describe the methods of operation of an oil-sealed rotary vacuum pump and an oil diffusion pump.

C.  Describe very briefly: 1) the McLeod gauge, 2) the ionization gauge, 3) the Phillips gauge, 4) the thermocouple gauge.  Which is an absolute pressure gauge?

## Possible Procedure Check List

### FIRST WEEK

- Turn on vacuum system (make sure you have read Appendix A).

- Evaporate trial silver film on to a clean glass slide.

- Turn off vacuum system.

- Practice loading test sample in to holder and make preliminary H and  measurements.

### SECOND WEEK

- Turn on vacuum system and load Ag wire into filament.  Install two cleaned glass slides.

- Evaporate final sample as well as the film thickness measurement sample at the same time.

- Take Hall measurements.

- If (3) is successful, make film thickness measurement on the glass slide and/or sample .

- Turn off vacuum system.