Appendix B:  Notes on using the program ACQUIRE

## Introduction

The program ACQUIRE is a customized data acquisition "virtual instrument" (VI) application written with LabVIEW (National Instruments) software. It provides control of the data acquisition card in the PC and allows display and saving to a file of the data buffers.  Voltages from the picoammeter analog outputs or other hardware are sent through a computer interface box, to a data acquisition card that resides inside the PC.  Below is a brief description of the various components of the data acquisition system and instructions for use of the software.

## Experiment to Computer Interface

The computer measures the voltage of electrical signals. The necessary electrical connections from your experiment are made to what is referred to as the “computer interface box”. Measurements can be performed on one or two different voltage signals. The connections between the experiment and the computer interface module are done in the following way:

- To measure a single electrical signal, connect it to channel 0 on the computer interface box.

- To measure simultaneously two different electrical signals, connect them to channels 0 and 1.  If the purpose is to produce an XY graph, connect the X signal to channel 0 and the Y signal to channel 1.

An extra electrical signal can be used to start (trigger) the acquisition of data.  That signal should be connected to the external trigger terminals of the interface module. It is also possible to start the data acquisition by pressing the red knob labeled EXTERNAL TRIGGER on the interface module.  The signal voltages are connected by shielded cable to the analogue inputs of the data acquisition card in the PC.

## Generalities and Starting the program ACQUIRE

The program ACQUIRE will allow you to

- Perform a sequence of measurements separated by a specific interval of time;

- Display a graph of the measurements taken, and make a hardcopy.

- Save the data to a disk file that can then be ported to another program for further analysis.

- Perform a Fast Fourier Transform of the data. This is useful as a diagnostic of noise.

To start the program, double-click on the icon on the desktop labeled ACQUIRE.  This will open the program. You will see a panel open with several buttons that will guide you through setup of the data acquisition channels, the graph, and acquisition of the data. To actually begin running the program, click on the arrow in the upper left corner of the screen (or type CTRL-R).  You will see the little STOP sign immediately to the right of the arrow turn RED (If you were to click on the red STOP sign, the program would stop running).

### Analog Channels

Using the drop-down menu in the area labeled Analog Channels, select the number of channels you would like to acquire. If you have only one signal, choose channel 0. If you have two signals, then select channel 0 to channel 1 from the drop-down menu by holding down the shift key and highlighting both items in the menu list.  At present, the computer interface box can only send 2 channels to the CIO-DAS08, although there are 8 available channels on the board.

### Analog Scale

Pull down the drop-down menu labeled Analog scale. This item permits selection of the range of voltages that can be measured without saturation (voltage outside the acceptable range of values). The default range is from 10 to +10 volts. This range should accommodate many measurement situations. If another range is needed, click on this item and choose from the list in the drop-down menu. For bipolar ranges X to +X, the numerical values produced by the analog to digital (A/D) converter and recorded in the memory of the computer range between 2048 and +2048. For unipolar ranges 0 to X, these values range between 0 to +4096. Choosing the appropriate analog scale allows you to match the resolution of the A/D to the voltage signals from your experiment.

NOTE: There are two boards used in these laboratories, the Keithley DAS-8 PGH and the Computerboards, Inc. CIO-DAS08-PGH (a less-expensive copy of the Keithley board).  They have different available programmable analog scales.  Each station has a version of Acquire matched to the card and there should be no need to make adjustments in the scales. However, it may be that the available scales differ from one lab station to another. Either board has 8 channels of 12-bit analog input, with a programmable analog scale.

### Sampling Frequency and Number of Points

These two items permit selection of the number of measurements to be performed every second (sampling rate), and the total number of measurements to be performed on each channel.  You should choose these two parameters in an appropriate combination according to the experiment.  Typically this is a tradeoff between how much data you want to try to digest (and how long you want to wait for the acquisition) and how rapidly the signals in the experiment are changing.  For example, you may wish to collect 1000 data points in a total time of 2 seconds. Then you would choose a scan rate of 500 Hz, and 1000 samples/channel.

You may choose a sampling rate from 15 to 8000 samples/second. There is no limit on the total number of samples that could be written to a file, but the file size will obviously increase with the total number of samples.  For practical reasons you should limit yourself to less than 10,000 samples.  The maximum number of points that can be displayed in a plot is 32768.

If the sampling rate is  300 Hz then the display plot will be live and will show the data as it is collected.  At higher rates, all the data is collected before it is plotted (a delay of a few ten's of seconds).

NOTE: the sampling rate that you choose is not precisely the sampling rate of the data acquisition. The true sampling rate is determined by dividing down a 1 MHz clock signal on the data acquisition board, but the division is integer division.  For example, if you select 3000 Hz as the scan rate, the true rate will be 3003 Hz. The true rate will be correctly returned after data acquisition begins. You will need to know the true sampling rate if you need the absolute time at which an event occurs within your data acquisition scan. The true sampling rate is displayed on the right portion of the screen in a box labeled Act. Freq.

### Trigger

If you wish to have data taking to start automatically as soon as you click on TAKE DATA, choose INTERNAL for the trigger setting. See the drop-down menu labeled Trigger, under the analog scale. If you would like to control the start of data taking through the computer interface box, choose EXTERNAL. In this mode, data taking will start when the signal going into the TRIGGER of the computer interface box makes the transition from 0 to 5 V.  Once you have set up all of the above parameters, you are now ready to begin taking data. Click on the button labeled TAKE DATA to start acquisition.

### Graph

After data taking has stopped, the channels in which you have collected data can be plotted in the graph at the bottom of the page. Click on the button PLOT DATA.  If you have acquired 2 channels of data, you can also make an X/Y plot of the two channels. Use the drop-down menus in the upper left portion of the graph to choose what you would like to be displayed.  Change the scales on the graph either by double-clicking on the lower and upper limits on each axis, or by using the buttons provided in the upper left. You may also change the appearance of the graph by right-clicking on area labeled “Plot 0”.  When you are satisfied with the plot, make a hardcopy by going to File/Print Window from the main drop-down menus.

NOTE: When you have two channels of data, the acquisition in the two channels is not quite simultaneous (only one measurement at time can be processed by the software). In order to make the measurements as simultaneous as possible, the sampling rate is doubled from whatever you have selected, and then every other sample is taken into each channel. For example, if you have chosen a sampling rate of 1000 Hz, the ACQUIRE program will actually use 2 kHz as the sampling rate, the first point will go to channel 0, the 2nd to channel 1, the 3rd to channel 0, etc. Thus the sampling rate in each channel is still 1 kHz and the measurements in channel 0 and channel 1 would be separated in time by 1/2000 Hz or 0.5 msec. This introduces a small error in the simultaneity of the data, but allows for fast acquisition.

### Saving Data to a File

Click on the SAVE DATA button to save data to a file. The data are saved in tab-delimited ASCII format in 5 columns (time, channel 0, channel 1, channel 2, and channel 3).  The files should be ported to some other program for further analysis (Excel, Mathematica, etc.).  A usual Windows dialog box comes up after the SAVE DATA click. You may append data to an existing file or write data to a new file. Note that you should write either to a floppy disk or to the C:\SCRATCH area on the local drive.

### Shutting Down the Program

When you have finished with the program, click on the STOP button to stop the program. Then from the main menus, use File/Exit to shut down the program altogether.