# Determining Planck’s Constant from the Photoelectric Effect

## References

Jenkins and White, Fundamentals of Optics, pp. 611-614

Eisberg, Fundamentals of Modern Physics, pp. 76-81

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- Make a sketch of current vs. voltage for the photocell; indicate clearly the polarities for both current and voltage.  Show the region in which saturation occurs.  Show clearly the region in which you will be taking data.

- Why does one not need to know the contact potential between the meter and the photocell?  (Explain in terms of the procedure used to obtain h from the data).

## Overview

Shining a light upon a metal surface can induce the emission of electrons.  It was known from experimentation that the only delays in this process are of the order of 10-8 second.  Classical radiation theory predicts delays of 107 seconds.  In 1905 Einstein postulated a particle theory of light as explanation, which was proven experimentally by Millikan in 1921.  Planck’s constant, the ratio between the energy of the emitted electrons and the frequency of the incident photons, will be measured in this experiment.

## Theory

Electrons are bound to materials by Coulomb attraction.  The energy required to remove an electron from an atom is termed the ionization potential.  The binding energy of an electron in a block of material will be different than that of a single atom, as the outermost electrons of the atom will no longer be occupying orbitals of the isolated atom but will be in states of the solid, no longer localized near one atom.  The energy necessary to remove an electron from a solid through a surface also depends on the conditions of the surface.  The energy required to bring an electron completely out of a sample, through a surface, is termed the work function of the solid.

When an electron has absorbed energy equal to the work function, it can escape from the material.  In the photoelectric effect the electron receives this energy from electromagnetic radiation shone upon the material.  According to the classical wave theory of light, the energy in the incident beam arrives continuously.  For an object as small as an electron to absorb the energy of the work function (a few eV) from light as bright as sunlight (~ 10-1 W/cm2) would require ~107 seconds.  This was not consistent with experiments that show the photoemission to occur after as little as 10-8 seconds.  Experiments also show that the photo-induced electron emission occurs quickly with even very low levels of blue light and will not occur with any level of red light.

Neither of these facts can be explained satisfactorily by the wave theory of light.  Einstein proposed that, rather than the energy being spread uniformly over the cross section of the beam, it was concentrated into small packets with a dimension on the order of the wavelength and with an uncertainty of position along the direction of propagation on the order of 300 cm ( = c  10-8 sec.).  Each packet, or “photon”, contains an amount of energy

E=hν ,  					IV-1

where  is the frequency and h is Planck’s constant (which Planck had used earlier as the separation of the levels of oscillators in materials giving off black body radiation).  Einstein also postulated that in the photoelectric effect the electrons absorb all the energy of one photon, and never receive just a portion of it.  Millikan was later able to show that Einstein’s hypotheses exactly described the photoemission experiments - that the photocurrent is proportional to the intensity of the incident radiation while the maximum energy of the emitted electrons is proportional to the frequency of the radiation.

In this model the emitted electron will have energy left given by:

Eleft = Eabsorbed – Eto get out    , 			IV-2

Eabsorbed = Ephoton = h.  The work function for an electron just under the Fermi surface is designated .  Electrons further down in the energy level will require extra energy, W, to get to the Fermi surface.  This gives

Eleft=hν−φ−W   				IV-3

For monochromatic light  will be constant.   will be constant for a given surface of a particular sample.  W can vary with the depth where absorption occurs.  This means that the emitted electrons can possess energies in the range

0<E<hν−φ . 				IV-4

## Procedure Outline

To determine h in this experiment you will be measuring Emax = h for several known frequencies.  The maximum kinetic energy will be determined by measuring the electric field that will just stop the most energetic electrons.  (Note that this measurement is sometimes performed with an accelerating voltage assisting the ejection of electrons rather than with a retarding potential as we do here.)  Our setup is shown schematically in Fig. IV-1.

Figure IV-1

When the retarding voltage is zero the full photocurrent is measured.  As the voltage is increased the less energetic electrons are unable to make it to the collecting anode.  Finally, when the retarding potential is equal to the maximum energy, no current is expected.  Thus, we expect from this simple model the curve shown in Fig. IV-2.  (Note that such a curve would have to result from a fortuitous combination of energy distribution of electrons and collection efficiency as a function of retarding voltage.)

Figure IV-2

The actual curves measured will also be complicated by two secondary effects.  There will always be some slight emission from the collecting anode.  The retarding voltage will be an accelerating voltage for these electrons and will collect them all.  Thus, this reverse current will be a constant.  The measured curves will actually appear in Fig. IV-3, with a saturation at Ic, the collector emission current.

Figure IV-3

The saturation current will increase slightly with voltage due to leakage current conducted through the remaining gas in the tube.  The second complicating factor is that the voltage recorded by the voltmeter is not exactly the potential between the photocathode and anode.  Connection of dissimilar metals gives rise to a potential difference known as the contact potential.  Thus, there is a constant offset in the measured voltage.  We will not need to find this contact potential, as we will not be determining the work function.  A constant offset will not affect the measurement of the slope, h.

In this experiment, the voltage necessary to stop the most energetic electrons will be measured for several wavelengths.

## Procedure Detail

The photocell used in this experiment (RCA 935) has a semi-cylindrical photocathode coated with an alloy of cesium and antimony (Cs3Sb).  The collecting anode is a wire placed along the axis of the cathode.  This photocathode has a high quantum efficiency so that a relatively high photocurrent is obtained with a given intensity of illumination.  It also has a low work function and responds even to red light.  The glass envelope of the tube is made from UV transmitting glass so that measurements can be made down to almost 2000 Å.

Since the monochromator is not calibrated and since the mercury lines are discrete, you should use the wavelengths listed below.  The source of radiation is a low-pressure mercury discharge lamp, producing lines at:

5780	Å	(yellow)

5461	Å	(green)

4358	Å	(blue)

4047	Å	(violet)

3660	Å

3132	Å	ultraviolet

2967	Å

A monochromator (Jarrell-Ash 82-410) is used to select one of the mercury lines at a time.  This device uses a movable diffraction grating to separate the light and selects the desired component with the exit slit.  The construction is as indicated in Fig. IV-4.  This arrangement, known as the Ebert mounting, is common in modern spectrographs and allows a given optical length to be placed in a box half that long.  When using the monochromator with the 6000 Å grating, the indicator on top will show the approximate wavelength of the first order spectra in nanometers.  Note that this reading is approximate, since harsh treatment of the device can cause slippage of the indicator shaft.  You should always adjust the position about the expected value to maximize the photoelectric current at zero voltage.  Note also that the lines of the second and third order spectra will occur at a reading of two or three times their wavelength.  If you have any trouble finding the mercury lines with the phototube, you may want to remove the tube and look for the visible lines by eye, to satisfy yourself that the monochromator is properly functioning.

Figure IV-4

You will be using a picoammeter to measure the photocurrent.  This is a very sensitive electronic device and must be treated very carefully.  Keep it on the least sensitive scale unless measuring, at which time you should step the sensitivity up until you get a reading.  It is difficult to measure currents this small.  There will be random fluctuations on the lowest scales that can be a significant noise to your measurements.  You must minimize all motion near the coax lead to the picoammeter, to avoid introducing measurable currents.

In this experiment, a computer will be used to sample the voltage output of the picoammeter.  A retarding voltage is applied to the phototube by a ramping power supply.  The value of the retarding voltage needed to reduce the phototube output current to zero, together with the value of the wavelength of the mercury line selected, permit determining Planck’s constant.

The computer will permit you to display a linear or logarithmic graph of the phototube output and also (for calibration) a linear graph of the ramping voltage as a function of time.  These graphs will be used for qualitative and quantitative analysis.

### Connections to record the phototube output:

The following connections will permit you to record the phototube output as a function of time and to determine for which computer sample the phototube output is reduced to “zero”.  The connections are made from the phototube to the picoammeter and to the computer interface box as shown in Fig. IV-1, as follows:

- Connect the ramp power supply positive terminal to the phototube cathode terminal using a BNC shielded cable.  It is suggested that the maximum supply voltage be initially set at 4 volts and adjusted as necessary.  Never go above 8 volts on the supply voltage!

- Connect the phototube anode terminal to the picoammeter using a BNC cable again.

- Connect the analog output of the picoammeter (at rear of the instrument) to the input of a low-pass RC filter (try 22 k and 10 fd and adjust to get the best data).  You may not need this RC filter in your circuit but, if you use it, be aware that it could produce unwanted time constant effects.  You should check that there are not any unwanted time delays in your data.  When computing the real time constant of the filter circuit, remember that there are 10 k in parallel with the signal terminals inside the computer interface box.

- The output of that filter is connected to the signal terminals at the computer interface box.

- Connect the trigger output at the ramp power supply to the trigger terminals at the computer interface box.

### Preliminaries:

- Put the ramp/reset switch on the ramp power supply in the reset position.  In this position, the voltage output is zero.

- Select the desired wavelength of the mercury source.  Fine-tune the spectrometer adjustment to obtain the maximum output at the picoammeter.

### Computer acquisition of the phototube signal:

- You are now ready to run the computer program “ACQUIRE” which will permit you to record the output of the phototube signal.  The details of how to use this program are described in APPENDIX B.

- For this experiment, it is suggested that you select a sampling rate of 50 samples (measurements) per second.  The total number of samples should be such that the decreasing exponential reaches within a few percent of the final expected value (estimated visually).  This may require a total sampling time of about 40 seconds or more and would imply a total number of samples of about 2000.  Trial and error will be helpful here.

- In this experiment, the trigger that initiates the taking of data will be external and provided by the ramp power supply as the reset/ramp switch is set to the ramp position.

- Gain code: You may select initially the -5 to +5 volt range and modify it later if desirable.  In particular, for certain wavelengths where the intensity is weak, you will find it useful to use a higher sensitivity.  In all cases, a bipolar range is necessary since positive and negative values occur.

- Set the reset/ramp switch on the ramp power supply to the ramp position.  This will initiate the ramping of the voltage of the power supply and the taking of the data by the computer.  (It is a good idea to put the reset/ramp switch to the reset position as soon as the data sampling is completed to insure a proper zero start for the next data sampling.)

### Calibrating the ramp voltage:

To convert the sample number (or time) at which the phototube output is zero to a voltage, you must record the voltage output of the ramp power supply as a function of computer sample number.

Disconnect the circuit completely without changing the settings on the power supply.  Connect the output from the ramp power supply to the input of the computer interface box.  Connect the trigger also.  Now take data for the same period of time as in the previous recordings.  You may also want to perform a second calibration curve with the RC filter in the circuit in order to check if the filter is contributing to any systematic errors.  The calibration curve obtained with the RC filter should just be shifted by an amount equivalent to the effective time constant of the RC filter.

Now you must measure a “stopping” voltage and relate it to the wavelengths to find h.  Since the traces smoothly roll over, there is actually no definite voltage at which all the electrons are stopped.  Since a constant offset in the stopping voltage will not affect the determination of h, any consistent measure of those voltages will serve.  Melissinos (page 24) suggests two methods.  Past experience has consistently shown that Planck’s constant measured using the method of intersecting tangents is often as much a factor of two less than the real value.  Better results are found by estimating the point at which the photocurrent vs. stopping voltage curve becomes a straight line with increasing stopping voltage.  Some students have even developed their own other methods of finding this elusive stopping voltage, with varying degrees of success.

## Discussion Questions (to be answered in lab report)

A.  Why is the monochromator designed so that the light that falls on the grating is parallel?

B.  What are the numerical values of:

- the electron charge (e) in coulombs.

- the electron volt (1 eV) in ergs.

- the frequencies of the mercury lines in sec-1.

- the velocity of the most energetic electrons seen in the experiment.

C.  How can you be sure that your results are not affected by stray light and/or leakage currents in the photocell?