# The Charge to Mass (e/m) Ratio of Electrons

## References

J. Classen, Physik Z. 9, 762 (1908)

Segre, ed., Experimental Nuclear Physics, Wiley, 1953, Vol. 1, Part V - focusing spectrometers

Mechanics, Berkeley Physics Course, Vol. 1, McGraw-Hill, 1965

p. 104-106		B field acting on a charged particle

p. 109			magnetic focusing

p. 114-115		charged particle in crossed E and B fields

Smythe, W. R., Static and Dynamic Electricity, McGraw-Hill, 1950 p. 249-251, Child’s equation

Plonsey, R. and Collin, R., Principles and Applications of Electromagnetic Fields, McGraw-Hill, 1961, p. 431-441, behavior of charges in E and B fields

Shamos, M.H., ed., Great Experiments in Physics, Holt, Rinehart and Winston, 1959, Chap. 16 - Thomson’s paper on cathode rays

Reitz, J. R. and Milford, F. J., Foundations of Electromagnetic Theory, Addison-Wesley, 1960, p. 156-158, Helmholtz coils

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- Is it important to locate the filament at a position normal to the line joining the two slits?

- What percentage error in e/m would result if the electrons passing through the first slit were travelling in a direction 5 degrees from the normal?

## Overview

This measurement, performed in 1897 by J. J. Thomson, was fundamental in establishing the existence of electrons.  Before that experiment it was the almost unanimous opinion of German physicists that cathode rays were due to some then unknown etheric wave phenomenon.  Thomson’s experiments showed that “...cathode rays carry a charge of negative electricity, are deflected by an electrostatic force as if they were negatively electrified, and are acted on by a magnetic force in just the way in which this force would act on a negatively electrified body moving along the path of these rays...”  The value of e/m was found to be independent of the nature of the gas in the cathode tube and much larger than for the previously known maximum value, that for hydrogen atoms in electrolysis.  Combining these facts with evidence of the long mean-free-path of cathode rays in air, Thomson suggested that the “carriers of the charges in the cathode rays are small compared to ordinary molecules...”

In this experiment, the ratio e/m, which enters directly into calculations of the orbits of charged particles in electric and magnetic fields, will be measured by Classen’s method.

Figure VI-1

## Theory

Classen’s method involves measuring the radius of curvature of electrons in a magnetic field.  The apparatus is shown in Fig. VI-1, with a pair of Helmholtz coils providing the uniform magnetic field parallel to the slits.  Electrons emitted by the tungsten filament are accelerated toward the filament slit by a potential between the filament and the slit plate.  With the filament aligned properly the electrons will pass through the slit perpendicular to the slit plate.  The electrons will then experience a force,

F=−ev×B	,			VI-1

which will give them a circular trajectory.  This force corresponds to a centripetal acceleration

F=evB=mv2r  ,			 VI-2

where m is the electron mass and r the orbital radius.  The velocity of the electrons is found from the kinetic energy they acquired from the accelerating electric field,

eV=mv22	,			 VI-3

where V is the potential.  From these two equations the value of e/m can be calculated

em=2V(rB)2	 mks units   . 	VI-4

In this apparatus, the orbital radius, r, is fixed by the position of a second slit, with a Faraday cup to collect the electrons which enter.

## Procedure Outline

With the apparatus set up and evacuated, the filament is carefully heated to promote electron emission.  The accelerating voltage is then set to some value.  The magnetic field in the Helmholtz coils is then varied and a plot of collector current versus magnetic field is made.  A large peak will occur where the field corresponds to the trajectory that enters the second slit.  A series of these plots are made at different accelerating voltages.   The ratio e/m is determined from the dependence of the peak positions on the accelerating potential.

## Procedure Detail

The enclosed system mounts on an arm fixed to a cover plate bolted to the vacuum chamber flange, as shown in Fig. VI-2.  Open the vacuum chamber, remove and examine the electrode structure.  The edges of the slits are beveled so that the slit widths are independent of the approach angle.  Remove the slit and check that there are no breaks or shorts in the filament or collector circuits.  The filament is a 7 mil diameter tungsten wire clamped between two arms mounted on ceramic insulators.  A much-used filament is very fragile.  If a new filament is mounted, the wire must not be bent sharply and should be under sufficient, but not excessive tension, to prevent it from sagging while hot.  The slit plate position can only be varied slightly so the filament must be carefully positioned to be under and parallel to the slit.  The clearance between the filament and the slit should be set to about 4 mm.  The two slit openings should be set with automobile feeler gauges.  Record the slit widths and the distance between the slits.  With the slit plate remounted, the filament should be visibly aligned under the slit (shining a light from the direction of the cover plate may aid in checking this).  Check that the filament is electrically continuous and is not touching the slit plate with a multimeter.  Replace the electrode assembly in the vacuum chamber.  The wing nuts need not be extremely tight.

Figure VI-2

The vacuum chamber is evacuated by a standard two-stage mechanical and diffusion pump system.  Instructions for operating the vacuum system are given in Appendix A of this manual.  The lab technician will check your familiarity with the vacuum system before you start, and will obtain the liquid nitrogen for the cold trap.  Begin to pump down the chamber.

With the master power switch off, connect the circuit as shown in Fig. VI-1. The accelerating potential between the filament and the slit plate is provided by a D.C. high voltage power supply.  A voltmeter on the power supply is used to measure the potential between the vacuum chamber and the filament leads.  The collector current is measured with a picoammeter.  Some of the currents in this experiment are quite small and the picoammeter is sensitive and delicate.  Always keep the meter on scale.  When not in use it should be kept on the least sensitive scale.  Minimize all motion when making a measurement as any movements of body or cables can cause large capacitive fluctuations in the current reading.  The picoammeter cannot respond instantaneously to changes in input and when making a plot the magnet current must be swept slowly and smoothly.  Turn on the picoammeter and record the residual current with all power supplies off.

Turn on the magnet power supply and adjust it to give about 0.5 amp.  Measure this current using the voltage drop across a standard resistor.  Check the direction of the magnetic field with a compass to make sure that the electrons will circle in the proper direction.  A calibration for the Helmholtz coils is posted near the experiment.  Consider how any systematic error caused by the magnetic field of the earth can be canceled.

Figure VI-3

With the HIGH VOLTAGE switch OFF, turn on the high voltage power supply.  After some delay the panel meter will indicate a voltage.  Turn the HIGH VOLTAGE switch to NEG and observe the picoammeter readings at various settings of the HV power supply COARSE switch.

When sufficient vacuum (~5 x 10-6 mm Hg) has been established, set the VARIAC to zero and turn on the filament transformer.  The filament current is measured on the AC ammeter while the emission current is read from the DC microammeter.  The filament current should be set to give a measurable collector current greater than the background measured above (2.5 amps or slighly greater), without burning out the filament.  One method of doing this is to turn on the negative high voltage, select the lowest picoammeter range on which fluctuations are no longer measurable, and then turn up the filament current until a steady collector current background is seen.  Typically, the filament will burn out at about 2.9A.

You can take measurements using the computer program ACQUIRE.  The voltage drop (see Fig VI-3) across the standard resistor can be recorded in channel 0 and the current from the collector can be recorded in channel 1.  Since the input to the computer should not exceed ~ 5 volts, one may use a voltmeter and a picoammeter as buffers, respectively.  Two terminals from the resistor should go to the input of the voltmeter.  The output from the voltmeter should be fed into the computer interface box.  This may introduce an attenuation factor which you must determine.

The apparatus can be changed to attempt to remove systematics and improve resolution.  Be certain that the HV power is off before touching the apparatus.

The entire electrode structure can be installed upside down and the magnetic field reversed.  The filament current, slit widths, and filament- slit alignment can be varied to maximize resolution and signal-to-noise ratio.  The system can be brought up to atmospheric pressure to make these changes and quickly re-evacuated by following instructions contained in Appendix A of this manual.

## Discussion Questions (to be answered in lab report)

- Explain any changes in the plots taken with different filament currents and slit widths.  Can you simultaneously maximize signal-to-noise ratio and resolution?

- What measured parameter contributes most to the statistical error of the final calculated result?  How much would it need to be improved to make another parameter more important?

- How does a charged particle move in crossed uniform electric and magnetic fields?

- Why is the measurement made in vacuum?  How do the vacuum gauges work?

- The direction and magnitude of the electric field between the filament and the slit plate are not constant, and the emission current itself will affect the fields.  How will the electron trajectories be affected in the region where both electric and magnetic fields are present?