# The Franck-Hertz Experiment

## References

Melissinos, Experiments in Modern Physics, p. 8-17

Eisberg, Fundamentals of Modern Physics, p. 124-8

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- Why is the collecting anode made negative with respect to the grid?

- Why must the Franck-Hertz tube be operated at an elevated temperature?  What is the consequence of going to a temperature even higher than recommended?

- Do you expect the voltage peaks to increase or decrease with increasing filament voltage?  Why?

- Why are voltage maxima used to determine the energy of the excited state?

## Overview

Bohr’s quantum theory of the atom was supported for emission of photons by the regular structure seen in atomic line spectra.  The experiment by J. Franck and G. Hertz in 1914 confirmed the quantization of energy absorption for atoms excited by collisions with electrons.  Mercury atoms, bombarded with electrons of known energy, only absorbed discrete amounts of energy during the electron atom collisions.  This strengthened the acceptance of Bohr’s postulates as it implied that the energy levels were intrinsic to the atoms and not due to some special properties of photons.

## Theory

Two basic postulates of Bohr’s quantum theory of the atom were the existence of discrete energy levels and the proposition that atoms can only absorb or emit by amounts corresponding to the difference between two such energy levels.  Bohr’s theory was based on the regular characteristics of atomic line spectra (as are observed in Experiment II), but evidence was needed that the discrete energy levels were intrinsic to the atomic systems.  One of the most direct experimental confirmations of the quantization of the energy absorbed by the atom was the experiment of J. Franck and G. Hertz.  Mercury atoms were bombarded by electrons of known energy.  The electrons lost only discrete amounts of energy, corresponding to differences in atomic states.

The Franck-Hertz tube is designed to be plano-parallel so that the accelerating fields are nearly uniform.  In order for the electron beam to be approximately monoenergetic, the hot cathode sits behind a non-emitting diaphragm held at the same potential.  The length of the interaction region is chosen to be several mean-free-paths for electron-atom collisions, so that electrons crossing this gap will collide with several mercury atoms.  (The decelerating region between the grid and the collector electrode is small so that few collisions can occur.)

When an electron collides with an atom, several outcomes are possible:

An “elastic” collision.  As the mercury atom is much more massive than the electron, little energy can be transferred from the electron to the atom, and the scattered electron continues with almost the same energy but on a different trajectory.

An “inelastic” collision.  If the bombarding electron has sufficient energy it can raise the outermost bound electron of the mercury atom to a higher energy state.  In this experiment 99.9997% of the mercury atoms will be in their ground state, and almost all of these collisions will result in the mercury electron being raised to its first excited state.  The laboratory manual with the apparatus contains an energy level diagram taken from Grotrian (1982).  This shows the first excited state to be about 4.9 eV above the ground state, which is the amount of energy normally transferred from bombarding electron to atom.  Also in the manual are copies of curves taken from the Landolt-Bornstein tables giving the cross sections for excitation by electron impact.  Examine these to estimate the likelihood of collisions that will raise atoms to higher states.

A “superelastic” collision.  Extremely rarely will an electron encounter an already excited atom, de-excite the atom to its ground state, and thereby gain kinetic energy.  This process is too rare to be considered as an explanation for any effect observed by the experimenter.

The net effect of all the possible interactions is that the initially monoenergetic electron beam will have a variety of energies after passing through the tube.

## Procedure

The apparatus is shown in Fig. III-1.  The picoammeter, A2, will measure the current of electrons, which start with energy corresponding to accelerating voltage, V2, and pass through the interaction region to end up with more than the energy corresponding to voltage V3.  That is, it will not be reading the total electron current, but just that current of electrons which still possess some of their original energy when they finish traversing the tube.  You will be measuring this current while changing the accelerating voltage.

As the potential difference between the positive grid and the cathode is first increased from zero, no current is collected until the electrons have enough energy to overcome the slight retarding voltage.  Then the current “increases” (it is a current of negatively charged electrons so that it is actually a negative current) as the accelerating voltage is raised.

Once the accelerating potential reaches 4.9 volts, inelastic collisions become possible.  Those electrons which suffer such a collision will not have sufficient energy to overcome the retarding potential between grid and collector.

Figure III-1

A1	0-400 DC Milliameter

A2 	Picoammeter

V2	Voltmeter on Power Supply

V3 	Voltmeter

The current to the collector electrode will suddenly decrease.  As the accelerating potential is further increased, even those electrons that have suffered inelastic collisions will again have energy to reach the collector and the current will continue to rise.  Each time the accelerating potential reaches a multiple of the first excited state energy of the mercury atom, the current will decrease suddenly.  This gives rise to a trace as in Fig. III-2.

## Procedure Detail

The mercury vapor pressure is very sensitive to temperature.  Consequently, the temperature should be kept stable, near 180°C.  The oven temperature is regulated and read from a Thermologic controller.  Note that the desired temperature can be set on the controller.  The oven should be switched on early enough in the period to allow it to warm up and obtain equilibrium.  Do not operate the tube above 200°C.  Continue to monitor the oven temperature throughout the data-taking.  When the temperature seems to have stabilized, set the grid voltage, V2, to 30 or 40 volts.  Unfortunately, the Thermologic controller uses 60 Hz ac current to heat the oven and this can produce an unwanted 60 Hz modulation of the measured collector current.  If you find to much 60 Hz noise on your data, it is recommended to turn the temperature controller off just prior to starting a data acquisition run.  Remember to turn it back on after the trace is finished so that the oven temperature can return to the initial value.  You should note in your lab book the starting and final temperature of the oven and take any temperature drift problems into consideration when analyzing your data.

The filament is powered by a 6.3 VAC STEP-DOWN transformer.  Make sure that the resistance R1 is set at maximum (fully ccw) before connecting the transformer, to avoid a large current surge to the cold filament.

Figure III-2

With the accelerating voltage, V2, set at maximum, the current through the Franck-Hertz tube should be about 0.1-1 nA.  The filament current, determined by resistance R1, should be adjusted to insure this.  Note that the anode current is a very sensitive function of the filament current, and should be monitored occasionally.

In this experiment, a computer will be used to sample the voltage output of a picoammeter.  The picoammeter is used to amplify the current generated by a Franck-Hertz tube.  A variable acceleration voltage is applied to the tube by a ramping power supply.  The values of the voltage at points where the current decreases due to inelastic collisions of electrons with mercury atoms are used to deduce the energy of the first excited state.

The computer will permit you to first display a linear graph of the tube current output vs. time  (proportional to acceleration voltage) and then a linear graph of the ramping (acceleration) voltage as a function of time for purposes of calibration.  These graphs will be used for qualitative and quantitative analysis.  It will be possible to save the data to a file for further analysis, or to print out graphs or numerical values.

### Connections to record the Franck-Hertz output:

The following connections will permit you to record the tube output as a function of time and to determine for which times the tube current begins to decrease (current peaks).  The connections are made from the tube to the picoammeter and to the computer interface box as shown in Fig. III-1 and in the following way:

- Connect the ramp power supply positive and negative terminals to the grid and the cathode of the Franck-Hertz tube.  The cathode connection is actually the low voltage side of the filament connection.

- Connect the Franck-Hertz collector terminal to the picoammeter using a BNC cable.

- Connect the analog output of the picoammeter (at the rear of the instrument) to the input of the computer interface box .  If there appears to be to much 60 Hz noise on your data, the variable resistance and capacitance boxes supplied can be used to construct a low-pass RC filter.  When estimating the correct values of R and C to use for the filter, it may be helpful to know that there is a 10 k resister in parallel with the computer interface box inputs.

- Connect the trigger output at the ramp power supply to the DC blocking interface box.  The output of that box is connected to the trigger terminals at the computer interface box.

### Connections to record the ramp voltage (for calibrations)

The following connections will permit you to record the voltage output of the ramp power supply as a function of time and to convert the time dependence of the current peaks to the voltage required to produce these peaks.

Using the same settings on the ramp power supply as used to measure the Frank Hertz output, connect the output of the ramping power supply to the input of the 0.1X voltage divider.  You should check that this voltage divider produces an output voltage 10 times smaller than the input voltage.  The output of that divider is connected to the signal terminals of the computer interface box.  If you connect the ramping power supply directly to the computer interface, the A-D card will be destroyed!

### Preliminaries:

- Put the ramp/reset switch on the ramp power supply in the RESET position.  In this position, the voltage output is zero.  One voltage ramp is generated by putting the switch in the ramp position.

- Adjust the maximum voltage produced by the supply to its maximum.  (This is approximately 40 volts.)

- Adjust the ramp speed for the ramping time to be approximately 20 seconds.

D. Computer acquisition of the signal using the program ACQUIRE:

- To start the program, double-click on the icon on the desktop labeled ACQUIRE.  This will open the program. (See Appendix B for more details on how to use this program.)  You will see a panel open with several buttons that will guide you through setup of the data acquisition channels, the graph, and acquisition of the data. To actually begin running the program, click on the arrow in the upper left corner of the screen (or type CTRL-r).  First, proper sampling rate and total number of samples should be selected.  For this experiment, it is suggested that you select a sampling rate of 50 samples (measurements) per second.  The total  number of samples should be such that total sampling time is equal to or larger than the power supply ramping time.  This would imply a total number of samples of the order of 1000.  Trial and error will be helpful here.

- In this experiment, the trigger will be external and provided by the power supply as the reset/ramp switch is set to the ramp position.  Go to the drop-down menu labeled Trigger, under the analog scale and choose EXTERNAL.

- Gain code: Using the drop-down menu labeled Analog scale, you may select initially the 0 to +1 volt range and modify it later if desirable.

- Once you have set up all of the above parameters, you are now ready to begin taking data. Click on the button labeled TAKE DATA to start acquisition.  This will ready the computer to start taking data upon receiving the external trigger signal.  Then set the reset/ramp switch on the ramp power supply to the RAMP position.  This will initiate the ramping of the voltage of the power supply and the taking of the data by the computer.  (It is a good idea to put the reset/ramp switch to the RESET position as soon as the data sampling is complete to insure a proper zero start for the next data sampling.)

- After data taking has stopped, the channels in which you have collected data can be plotted in the graph at the bottom of the page. Click on the button PLOT DATA.  If you have acquired 2 channels of data, you can also make an X/Y plot of the two channels. Use the drop-down menus in the upper left portion of the graph to choose what you would like to be displayed.  If the data looks good, you may save it to a file by clicking on the SAVE DATA button.  Before you continue, it would be a good idea to make sure that your data is actually saved on your floppy disk or on the C:\SCRATCH area of the local drive.

The procedure to make a graph of the voltage of the power supply as a function of the computer sample number will be the same.  It is important to use the same number of samples per second and total number of samples.  If you used an RC filter in your circuit, it is also important to understand the effect of the RC filter on your data because that circuit could introduce a time delay which alters the voltage-versus time in the measurement of the tube output.  Also, depending upon the choice of R, the RC filter, the 10 k protection resistor together with the input impedance of the A-D converter can actually behave as an unwanted voltage divider.  You need to know what your measuring circuit is doing to your data and include these effects in your data analysis.  You will reduce the ramping voltage by a factor of ten with a voltage divider so that the 40 volt ramping voltage range can be accommodated by the computer (10 volt maximum).  Multiply the recorded value by ten (or your measured value of the attenuation factor) to obtain the correct ramp-voltage verses time conversion.  It should be possible to take both the power supply voltage and output current at the same time using channels 0 and 1.  Remember to have the 10/1 voltage divider in the leads connected to the A-D converter.  Verify that this dual signal acquisition set up does not cause a measurement problem.

Measuring small currents always calls for care.  The picoammeter is delicate and should always be kept on scale.  Keep it on its least sensitive scale when not using it.  You must minimize movement near the circuit when measuring as you can induce currents even in shielded cable.  Note that the thermologic temperature controller may radiate noise into the system.

Determine if the shape of the curve depends upon the value of V3.  Make an enlarged plot covering two successive maxima for two rather different values of V3.

Determine the energy of the first excited state of mercury.  Successive maxima of the collector current, A2 should occur at voltages:

Vmax(n)=Vxn+b .

Since the background current rises, in order to find the peak positions one should first fit the background with a low order polynomial, and then fit individual peaks to a gaussian using ANALYSIS or some other similar program.  The center of the gaussian fit obtained from the fit statistics will give the position of the peaks.

The form of this equation should suggest how to determine Vx from your measured values of Vmax.  What is the physical significance of b and why do you fit the positions of the maxima rather than the minima?

## Discussion Questions (to be answered in lab report)

A.  What is the maximum fraction of its energy that an electron can give to a mercury atom in an elastic collision?

B.  What is an electron-volt and what is its value in ergs?

C.  What happens to the excited mercury atoms?  If radiation is involved, what is its wavelength and how could it be detected?

D.  The width of the first excited state in mercury is quite small (0.001 eV).    What causes the much larger observed widths of the current minima?