# Measurement of the Velocity of Light

## References

Griffiths, Introduction to Electrodynamics, (Prentice-Hall, 1999) Chapter 9.

Serway, Physics for Scientists and Engineers, Vol II, (Harcourt Brace, 1996) Chapter 36.

Jenkins and White, Fundamentals of Optics, (McGraw-Hill, 1-8 (1976)) pp. 403-421.

Hecht, Optics (Addison-Wesley, 1998) p 158 ff.

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- Will you be measuring the group or the phase velocity in this experiment?

- Show that the intensity, I2, returned to the observer is a maximum when the reflectivity of the half-silvered mirror is 50%.

- Draw diagrams tracing the light from the laser through the various optics to the eyepiece for the cases: 1) rotating mirror stationary, and 2) rotating mirror in motion.  How many images are visible at any one time?

## Introduction

If light is an electromagnetic wave it can be represented by periodically varying electric and magnetic fields.  Thus

Ex=E0sin2πft−zλ				I-1

Hy=H0sin2πft−zλ				I-2

may represent a monochromatic beam travelling in the positive z direction.   The radiation has a unique frequency f and wavelength , where f is the number of wave amplitude maxima passing a fixed point per second and  is the instantaneous separation of successive wave amplitude maxima.  The velocity with which any phase angle   travels is just  = f  and is known as the phase velocity.  In a vacuum all frequencies travel with the same phase velocity c, while in a medium of refractive index n(f) the phase velocity is

v(f)=cn(f) . 				I-3

In practice no beam is monochromatic but contains radiation with a certain spread of frequencies f  about the frequency f0.  Thus the electric field associated with a real beam becomes

Ex=(f0−Δf)(f0+Δf)E0sin2πft−fn(f)zcdf	. 		I-4

For any spectroscopic line source the limits f over which the amplitudeE0(f) has non-zero values is very small; i.e.,

Δff0<<1  . 					I-5

The general form of Ex associated with any real beam can readily be seen by considering a beam consisting of only two frequencies f0 + f and f0 - f  with equal amplitude, then

Ex=2E0cos2πTt−zcΔf(nΔf−fΔn)sin2πf0t−zλ0  ,		I-6

or		  Ex=2E0cos2πTt−zusin2πτt−zv  , 			I-7

where v is the phase velocity:                    v = f0 λ0 =cn  ,			I-8

and where u is the group velocity: 	    u=cddf(nf)  .			I-9

Thus in any real light beam there is a term sin2π(f0t−zλ0) which represents variations at the carrier frequency f0 and a term E0cos2πTt−zu which represents amplitude modulation.   τ=1f0 is the period of the carrier frequency oscillation;

T =1Δf is the period of amplitude modulation and shows the spread of frequencies in the beam.  The energy density in the beam is proportional to the amplitude squared

I=Ex2  ,					I-10

and the velocity with which energy is transported is the velocity with which Ex(max) travels, therefore Tτ, and the velocity with which energy is transported is the group velocity u.

In vacuum u =v =c, while in most common substances uvc for frequencies in the visible spectrum.  In certain cases either u or v or both may be greater than c; however when uc, it can no longer be interpreted as the velocity with which energy propagates, i.e., the simple analysis given above no longer holds.

Any experiment to measure the velocity of light must measure the group velocity u at which energy is transported over a given distance.  There is no known method of measuring directly the phase velocity v.

Many methods have been developed for determining the group velocity of light; we shall use the method first successfully applied by Foucault and Fizeau in 1850 after being suggested to them by Arago.  As with all other methods, this method consists essentially of timing the passage of energy from a light source to a distant mirror and back to the light source.

## Apparatus

The layout for the apparatus is shown schematically in Fig. I-1.  The plane of the diagram is the horizontal plane; in practice the apparatus is also displaced in the vertical plane so that M1 and M2 fall above the laser and R respectively.  This allows the whole experiment to be mounted on one wall of the laboratory. M1 and M2 are plane mirrors.  R is a one-centimeter diameter flat mirror that can be rotated at high speed.  It is supported by high speed ball bearings, mounted in a protective housing, and driven by a mylar belt connected to a pulley and a DC motor.

Figure I-1

Note that the mirror appears to have two flat sides but only one is shiny.  The shiny side is the one you want to use for alignment.  The rotational speed is set and read with the motor controller placed near the observation point.  All alignment is done with R stationary and the motor controller off!  L1 is a spherical lens with a focal length of approximately 5 m, and G is a pellicle beam splitter.  This splitter is fragile and expensive.  KEEP YOUR FINGERS OFF!  S is an adjustable vertical slit, normally set to a width of approximately 0.2 mm.  A 2.0 mW He-Ne laser is used as the light source.  The wavelength of the red light is 6328 Å.  The position of the beam from the laser can be adjusted both vertically and horizontally with the alignment device in front of the laser.

## Setting-up Procedure

With the mirror R stationary and off, align the optical system so that light from S traverses the pellicle beam splitter G, and after reflection from R is focused by L1 on mirror M2 via mirror M1.  You should start with the slits in front of the laser wide open.  Make sure the beam is going through the center of the slits and also through the beam splitter.  The angle of reflection from R can be adjusted by slightly rotating the pulley which the mylar ribbon is attached to using your finger.  Note that there is a lens directly in front of the mirror and you must make sure that the reflected light from this lens does not trace back to the beam splitter.  Thus the first image S1 of S appears on the surface of M2.  Since M2 is a mirror, light from this image can be reflected back to M1, L1, and R, so forming the second image of S, S', coincident with S when R is stationary.  The image S' can be seen on the image plane by an observer making use of the pellicle beam splitter G.  When R is stationary, S' falls at E and can be seen with a small magnification (10) through the microscope eye piece.  To avoid excessive exposure to the laser beam while viewing through the microscope with the rotating mirror stationary, adjust the variable slit to a minimum width.  Note that if R is moved slightly by hand, the image S1 moves across mirror M2 but the image at E remains stationary.  Before attempting to set up any of the optics, determine the approximate positions for each piece using the simple lens formula and the above statements regarding positions of focus.  One further requirement must be met: because of the size of the laboratory, the distance L cannot be greater than 7 m.  The beam splitter G should make a 45° angle with the laser path in order for EE' to measure the beam splitting accurately.  Why?

Knowing the approximate positions at which each component must be placed, set up the optical system and adjust for optimum focus.  Verify the statement regarding the movement of S1 and E.  Note the asymmetric multiple nature of the image S1 and the symmetric multiple nature of the image at E.

Accurately measure the distances SR, RL1, L1M1 and M1M2.

## The Pellicle Beam Splitter G

G has a reflectivity R of approximately 50% and transmission T of approximately 50%.  There is negligible absorption.  Referring to Fig. I-2, if the intensity incident on G from the source S is I0, the intensity going on to the optical system is I1 and

I1=I0T	. 				I-11

The optical system can be assumed a perfect reflector for that part of the revolution of R during which an image is formed at E; thus we assume that the intensity returning from the optical system is also I1.  The intensity reflected from G into the image of E is I2 and

I2=I1R=I0RT   or    I2=I0R(1−R) .			I-12

I2 is a maximum when R ~ 50%, hence the choice of a half-silvered mirror or a pellicle beam splitter and not an uncoated piece of glass, as used by Foucault and Fizeau.

Figure I-2

## Theory of the Experiment

Light from S is reflected from R to L1, and M1 and M2 then back to R and so back to its origin S.  Because of the finite velocity of light u, the time taken to cover the return journey from R to M2 and back to R is

t=2(RL1M1M2)u (sec) ,			I-13

where		    (RL1M1M2)=RL1+L1M1+M1M2 (cm) .			I-14

If during this time the mirror R is turned through an angle  (radians), the returning beam reflected from R towards S will be turned through an angle 2.  Consequently the image of S at E will be displaced to the new position E', such that

EE'=2θRS (cm) .				 I-15

The distance EE' can be measured by the observer using the travelling microscope eyepiece.  The angle through which R turns in the time t depends on the angular velocity of R,  radian/s,

θ=ω t	 .				I-16

Therefore		      EE'=4ω(RL1M1M2)RSu ,			I-17

and the only unknowns in this equation are  and u.  The angular velocity  of R can be determined from the measured frequency of rotation f since

ω=2πf					I-18

Therefore			EE'=8π(RL1M1M2)RSuf			I-19

or				u=8π(RL1M1M2)RSEE'f .			I-20

Let RS and RS' intersect the beam splitter G at B and B', respectively.  Since we do not see the points E, E' on a screen, it is not quite correct to say that

EE' = 2θRS  .				I-21

If we assume that BS = BE, and if the line RS' intersects BE at the point C, then

BC = 2θRB . 				I-22

But, on the scope we notice EE', which is equal to CB'.  (This is because we do not have the beams projected on a screen, in which case the separation between two points on the screen will depend on the position of the screen.  In our case only the angular position of the scope matters.)  Hence

CB'/CB = tan(EBB')				I-23

CB' = tan(EBB')2⋅θ⋅RB			I-24

EE' = CB' = tan(EBB')2⋅θ⋅RB		I-25

Now using the expression for t, one can modify Eq. (I-20) to find u.

## Optimizing the Experiment

Three criteria must be borne in mind to optimize this experiment:

A.	The displacement EE' must be measurable to some desired accuracy, i.e., EE' must be larger than a certain value.

B.	The system must be so designed that sufficient light intensity passes through the system so that the displaced image E' is visible to the observer.  Try to minimize the diameter of the image spot for best measurement precision.

C.	We notice that EE' is proportional to  and to the product (RL1M1M2)RS.  Since the intensity seen by the observer in the image at E' is independent of the velocity once the mirror rotates at greater than about 25 rev/sec, it is clearly profitable to make f as large as possible.  f is limited by the equipment.  The rotation rate is reversible and continuously variable from 100 to 1000 rev/sec, and the maximum rate of 1500 rev/sec can be obtained by holding down the MAX REV/SEC button.  The motor is not stable when operated at a frequency below 200 Hz.  So while taking data, either the frequency range is to be limited to beyond 200 Hz or a larger uncertainty has to be associated with the frequency measurements below 200 Hz.

## Safety Notes

The laser has a 2 mW output, or one-fifth the brightness of full sunlight.  As with many items, danger exists only when it is mishandled or misused.  Read the laser safety information on the bulletin board in Room 3206 before starting.

A.	When aligning the apparatus, keep the door of the room closed to avoid shining the laser light on those passing down the corridor.

B.	Note that there is a small box attached by screws to the wall behind the travelling microscope.  This box is used to dissipate the light projected against the wall by the half-silvered mirror and whose reflection from the wall will interfere with the viewing through the microscope.

C.	Ear muffs are available to deaden the noise of the mirror motor at high speeds.  Laser goggles are also available.  (You will, however, be unable to see the light when wearing them).

## Experimental Procedure

The distance between the mirrors can be measured accurately by using the plumb bob to locate a point on the floor under the mirrors, and then by measuring the distance between the points using the steel tape measure.

The digital readout and frequency controls are located on the front panel of the motor power supply and controller.  The digital readout provides measurement of mirror speed to within 0.1% and 1 rev/sec.  The frequency is variable (clockwise or counterclockwise) from 100 to 1000 rev/sec; the maximum speed of 1500 rev/sec can be obtained by holding down the MAX REV/SEC button.  When the equipment has been aligned properly, turn on the rotating mirror to a frequency of 800 rev/sec.  Take data for 10 to 15 different frequencies in the clockwise direction, allowing the motor to stabilize at each setting before recording the frequency.  Do not allow the motor to run at very high or very low frequencies for very long because the motor driver may malfunction forcing you to turn off the power and wait until things cool down.  Repeat the measurements in the counterclockwise direction.  With the data of displacement versus frequency (plot ccw data as negative frequency), carry out a least squares fit to the data using a program such as LINREG.  The fitted slope of the line determines u and u .

NOTE: When reversing the direction of the travelling microscope, there will always be some movement in the knob before the carriage responds (backlash).  To avoid this source of error it is recommended that one adjust the initial position so that measurements are made always turning the carriage in the same direction.

## Discussion Questions  (to be answered in lab report)

- Show that for a simple convex lens of focal length f, the minimum distance between object and image is 4 f  when both the object and the image are real.  How could this fact affect the present experiment?  Calculate reasonable values for SR, RL1, L1M1, and M1M2 for the present apparatus and laboratory size.

- Calculate the object and image distances and give a ray diagram for a simple convex lens with f = 10 cm used as a magnifying glass with a magnification of 3 times.

- State the law of optics which tells us why as mirror R rotates slowly so that image S1 traverses M2, the image of S at E remains stationary.

- Is a monochromatic source necessary for the experiment?  If yes, why?  If no, why not?  What other parameters should be modified for both the cases?