# Introduction to Nuclear Physics and Gamma-Ray Spectroscopy: Part I

## References

Melissinos, Experiments in Modern Physics, p. 194-204.

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- What factors are to be taken into account in selecting the source-to-detector distance?

- Make a rough sketch showing how you would make a graphical determination of the half-life of 116In.  Explain exactly how you would extract the half-life information.

## Introduction

From the time of the Greeks until well into the 19th century, the atom was thought to be the smallest indivisible piece of matter (particle).  The discovery of the electron (J. J. Thomson, 1897) and the proton (Rutherford, 1919) led relatively rapidly to the idea that the atom consists of a positively charged nucleus with negatively charged electrons around it, this idea being finally formulated by Bohr in 1913 (see Experiment II).  Since the beginning of the 20th century, so much progress has been made in understanding the bonding of electrons to the nucleus to form an atom that it is reasonable to say that this part of the structure of the atom is fully understood, at least insofar as the many-body aspects of the atom can be solved.  Regarding the structure of the nucleus, however, no such complete understanding can be claimed.  Whereas the force holding electrons to the nucleus is known to be the electric force whose properties are well understood, the forces holding the protons and neutrons (Chadwick, 1932) together in the nucleus are not clearly defined.  Only recently has a fundamental theory in terms of quarks and Quantum Chromodynamics been advanced which has the potential of providing an understanding of the strong interaction.

The diameter of an atom (the “electron cloud” for an atom) is of order 10-8 cm, whereas the diameter of the nucleus is of the order 10-12 cm.  The energy binding an electron to the atom varies in the range 5 to 100 eV and is attributed to the attraction of positive and negative charges.  The energy binding a proton or a neutron to a nucleus is in the range of 1 to 10 MeV and the nuclear forces are not strongly charge-dependent.  Since these forces are never detectable between particles separated by distances much greater than the nuclear diameter, they must be very “short range” forces.

In 1914 it was discovered (by Rutherford and Andrade) that the positions of the nucleons in the nucleus are not rigidly fixed but that a nucleus may be distorted (excited), for instance, by bombardment with other particles, and when this “excited” nucleus returns to its “ground state” the excess energy may be released as a quantum of radiation (photon).  Thus it became apparent that the particles in the nucleus can absorb and emit photons in a manner similar to the bound electrons in an atom.  Just as the photon energy absorbed or emitted by an electron is a large fraction of its binding energy, so the photon energies emitted and absorbed by the nucleus are large fractions of their binding energies.  These photons are known as gamma-rays and typically have energies ranging up to 10 MeV.

Our detailed knowledge of the electronic structure of the atom stems very largely from careful studies of the spectrum of photon energies emitted by excited atoms (atomic spectroscopy); similarly, nuclear spectroscopy, including -ray spectroscopy, has played an important role in the development of our understanding of the nucleus.

Experiment VII consists of two basic parts.  The first part is an introduction to the principles and apparatus involved in a -ray detection system in which the response of a NaI(Tl) detector to a source of monoenergetic -rays will be studied.  A cesium source should be used to find the response of the detector to the particular source.  Energy calibration should be done using other sources listed in the table.  In the second part the half-life of a radioactive isotope produced by neutron irradiation will be measured.

## Detection

### The NaI(Tl) Scintillation Detector

The detector used in this experiment is an encapsulated, thallium doped, sodium iodide crystal optically coupled to a photomultiplier (Fig. VII-1) by a layer of optical grease between the face of the photomultiplier and the crystal.

When an incident -ray is absorbed in the crystal it interacts essentially totally with the atomic electrons, and a fixed fraction of its energy is converted into the production of photons of much lower energy.  The spectrum of these photons peaks at about 410 nm, which is within the spectrum of visible light (blue).  The processes for absorbing -rays are described in the next section.  The crystal is encapsulated in a form-fitting container with a highly reflective interior surface, which diffusely reflects light back into the crystal.  The photons produced in the -ray absorption may undergo one or more reflections at this surface until they eventually enter the photomultiplier (PMT) where they can liberate electrons from the photocathode through the photoelectric effect.  These photoelectrons cause a pulse of electric current that is then greatly amplified in the PMT, which serves as a low-noise, high-gain amplifier.  The pulse height of the output signal is proportional to the initial number of photoelectrons and is therefore also proportional to the energy of the incident -ray.  Clearly, the ultimate energy resolution depends statistically on the number of photoelectrons liberated.

Figure VII-1

### Energy Loss Processes

When a -ray photon enters the NaI(Tl) crystal (or any matter) it can interact in one of three ways: photoelectric capture, Compton scattering or pair production.  In the photoelectric effect all of the energy is absorbed by a bound electron and is converted into the kinetic energy of this electron (minus a small binding energy and recoil loss).  In the Compton scattering process, you will recall that the incident photons are scattered by electrons that may be considered as free, since the electron binding energy is small compared to the -ray energy.  The initial -ray energy, E , is shared by the electron, Ee, and the scattered photon, E´.

Figure VII-2

The amount of energy transferred to the electron is a function of the scattering angle, , and all energies from zero up to a certain maximum can be released in the crystal.

The difference in wavelength between the scattered and incident photons is

λ'−λ=hm0c(1−cosθ) where E=hcλ			VII-1

Solving for the energy of the deflected photon we get:

E'γ=Eγ1+Eγmc2(1−cosθ)				VII-2

and the energy given to the electron is:

Ee=Eγ−Eγ1+Eγmc2(1−cosθ)			VII-3

The maximum energy given to the electron (and hence to the scintillator) occurs when  = 180°.

These two processes (photoelectric and Compton) dominate up to about 2 MeV -rays.  The electrons released in these processes travel a short distance in the crystal and lose their kinetic energy by excitation and ionization.  A fraction of this energy then appears in the form of visible light quanta emitted when the atoms return to the ground state.  Since the number of excitations is proportional to the energy deposited in the scintillator, a measurement of the total light emitted serves to determine that energy.  The light emission occurs in a short interval (0.23 sec decay time) following the interaction of the initial high energy photon with an electron within the scintillator.  The light is reflected into the photocathode of the photomultiplier tube where the visible photons produce photoelectrons that cascade through the photomultiplier, increasing in number at each diode (Fig. VII-3).  The total charge reaching the anode produces a voltage pulse across the anode resistor.

The number of electrons which are collected at the anode is many orders of magnitude greater than the initial number of photoelectrons because of this multiplication process.  Suppose that every electron produced at a given dynode produces, on average, m secondary electrons upon impact with the next dynode in the chain.  The total gain in electron number in a tube with N-dynode stages is mN.  The average multiplication m, depends upon the inter-dynode potential difference.  For typical operating voltages the multiplication is about 3 to 4.  We will use the RCA 6655A photomultiplier in this experiment.  This is an N = 10 PMT whose current gain is 1.6x106 at 1000 volts.  This corresponds to a multiplication m  4.2.  In this experiment you will study the response of such a detector to 0.662 MeV -rays.  Due to light collection efficiency and the conversion efficiency of the photocathode, one obtains approximately 1 photoelectron per keV of energy loss by the -ray.  Assuming all of the photons reach the photocathode within 1/2 sec, the expected average output current at an operating potential of 1000 volts is

(0.622 MeV)103photoelectronsMeV1.6x10610−62sec−1= 2.1x1015 electrons/sec

The PMT is connected to an ORTEC Model 276 PMT base whose anode output has an internal impedance of 1 k.  The average potential of the output pulse is therefore

2.1x1015esec−1.6x10−19Ce103Ω = −0.34 volts

Figure VII-3

In addition to the signals produced in this fashion, there will also be a substantial rate of PMT noise pulses.  These noise pulses are generated when electrons are emitted from the photocathode because of thermal effects.  This thermal emission occurs whenever an electron acquires sufficient thermal energy to overcome the potential barrier which binds electrons in the material of the photocathode.  Evidently the rate for such emission increases as the temperature increases.  It also depends upon the PMT potential which gives rise to an electric field at the photocathode.  It is generally desirable, therefore, to operate the PMT at the lowest possible potential in order to reduce the noise rate.

The noise pulses are typically of much lower pulse height than the signal for single photoelectrons which is estimated at 1000 volts as

−1.6x10−19Ce1.6x1060.1 μsec−1103Ω = −3x10−3 volts

Fig. VII-4 shows the energy loss in the crystal due to the photoelectric and Compton interactions.  The data shown in this figure are obtained from the irradiation of the detector by -rays of a unique energy.  The resulting distribution of pulse heights exhibits three distinct features:

The photopeak corresponds to events in which the entire energy of the incident -ray is absorbed in the NaI crystal.  These events are mainly due to photoelectric capture of the -ray.  In this case the resulting electron stops in the crystal and yields all of its kinetic energy to atomic excitation.  Alternatively, the incident -ray undergoes an initial Compton scattering followed by the photoelectric capture of the scattered -ray.  Here again the entire initial energy appears as kinetic energy of electrons and this energy is converted as atomic excitation in the crystal.

Figure VII-4

The Compton scattering structure is due to Compton scattering events in which the scattered -ray escapes from the crystal.  In such cases only a fraction of the initial energy is converted in the crystal.  Note that the upper edge corresponds to 180° scattering, the maximum energy which can be transferred to the electron.

The backscatter peak corresponds to -rays which undergo Compton scattering inside of the radioactive source, or other nearby objects, and then enter the crystal.  At  ~ 180° for such events, the energy, E´, of these back scattered -rays is given by

E'γ=Eγ1+2Eγmc2			   	VII-4

For E = 0.662 MeV, E´ = (0.278)E = 0.184 MeV.  Since there are fluctuations in the light output of the crystal and because of the statistical nature of the processes occurring in the photocathode, the voltage spectrum is smoothed or “smeared out”.  This “smearing” is most clearly seen in the photoelectric peak which results from monoenergetic recoil electrons but is spread into a spectrum of signal amplitudes that appears Gaussian in shape.

For -rays above a threshold of 1.02 MeV pair production is possible, although it is not too important below about 2 MeV.  The -ray creates a positron-electron pair with total kinetic energy of E – 2mec2.  The positron is unstable and, after coming to rest, annihilates on an electron producing two photons each of energy 0.511 MeV.  If both of these annihilation quanta escape from the detector, the energy absorbed by the scintillator will be 1.02 MeV or 2 mec2 less than that of the incident -ray.  With the additional possibility that one or both of the annihilation quanta will suffer photoelectric capture within the scintillator, energies of E– 2 mec2, E – mec2, or E may be released in the material.  Peaks in the response of the crystal may be noted at each of the three energies.  The first occurs if both 0.511 MeV -rays totally escape from the crystal, the second if one escapes and the other is captured by a photoelectric event, and the third if both undergo photoelectric capture.  An example of such is presented in Fig. VII-5.

Figure VII-5

The 0.511 MeV -rays may, instead, undergo Compton scatterings in the crystal resulting in a distributed energy loss.  These effects are described in greater length in the references listed at the beginning of the write-up to Experiment VIII.

## Experiment

### Electronic Set-Up

The various components of the apparatus should be connected as shown in Fig. VII-6.  The connections are made using 51 coaxial cables (RG58) fitted with BNC connectors, with the exceptions of the H.V. cable, which is RG59 and the preamplifier power cable that is permanently connected.  The function of each component is as follows:

- The PMT Base and Preamplifier:  The photomultiplier tube is RCA 6655A connected to ORTEC Model 276 base.  The base supplies the dynode potentials in uniform steps from 0 at the cathode, to the full +H.V. at the anode.  These dynode potentials are tapped off of a resistor voltage dividing chain and are distributed to the PMT through the appropriate pins on the bottom of the tube.  The base has an output for the charge collected at the anode.  This ANODE output produces a negative pulse whose width is about 1/2 sec.  The base also contains a preamplifier which amplifies the charge collected on the 10th dynode.  The arrangement is shown in Fig. VII-7:

Figure VII-6

Figure VII-7

The charge-sensitive preamplifier is designed to provide a modest gain while maintaining a high signal-to-noise ratio.  This requires a large feedback resistor, Rf, which in turn causes a long decay time for the output pulse (~30 sec).  The preamplifier appears to the source as a large capacitor, Cf, and must in fact be much larger than the source capacitance.  The gain, A, is typically greater than 104.

- The Amplifier and SCA:  The ORTEC Model 490B includes both a low-noise shaping amplifier and a single channel analyzer (SCA).  The amplifier accepts positive pulses up to 12 volts.  The pulse is shaped into an approximately Gaussian shape and amplified by a continuously adjustable gain from 12 to 1280.  The AMP output is the output of the amplifier stage and is used when making the proper gain adjustment.  This output is linear in the range 0 to 10 volts.  The amplifier contains a “pole zero” adjustment.  This adjustment allows one to minimize the time it takes the pulse to return to zero, thereby minimizing pile up.  It should be adjusted before taking data.  Expand the vertical scale of the oscilloscope so that one can observe the tails of the pulses during the adjustment.

The SCA produces a fixed-shape output pulse whenever the amplifier output falls in the range V (lower) to (lower)+V (window).  Both V (lower) and V (window) are selectable from 0 to 10 volts using the potentiometers on the front panel of the unit.  The SCA output is a +5 volts square pulse that will drive the Multichannel Scaling input of the multichannel analyzer.

- The TEKTRONIX Type 2245A 100 MHz Oscilloscope:   Either of the two input channels (1 or 2) can be used to display the NaI pulses. Connect the signals to the appropriate channel and choose the display via the buttons in the section labeled MODE. The vertical gain can be adjusted independently for each channel. Turn up the READOUT intensity to see the vertical and horizontal scales across the lower portion of the screen.

The horizontal sweep can be triggered externally or by the pulse itself. Set the trigger select to A, the mode to NORM, the channel to either 1 or 2 (depending on which input you’ve chosen) and the coupling to DC. Choose the trigger slope to be positive or negative depending on the type of signal you’re looking at, and adjust the trigger level until a trace appears; the TRIG'D indicator will light whenever a pulse has triggered the display. The signal height and width can be measured using the MEASUREMENTS buttons and the TIME/POSITION cursors.

- Multichannel Analyzer (MCA) and its Use:  Measuring the spectrum of a source with a single channel analyzer set at a certain window width is obviously slow and tedious.  Furthermore, one would not obtain a true spectrum for a rapidly decaying sample.

A multichannel analyzer accepts a wide range of pulse sizes.  The pulses are sorted by size and stored in separate channels or bins of a memory chip.  The sorting is accomplished by an analog-to-digital converter (ADC).  In the operation of a typical ADC the pulse height of a particular signal charges a capacitor up to the peak voltage of the pulse.  The input signal drops quickly to its zero level but the capacitor is forced to discharge at a constant current.  The length of time for the voltage on the capacitor to discharge to zero is therefore proportional to the magnitude of the input pulse.  The length (time) of the discharge is digitized by counting pulses from a fixed frequency oscillator (100 MHz) during this period.  The number of counts recorded determines which of the many channels are to be incremented.  Typically, each channel has a capacity of approximately 106 counts.  A multichannel analyzer is therefore somewhat analogous to many single channel analyzers and scalars, each with the same window width, but successively greater baseline settings.

A multichannel analyzer can only evaluate one pulse at a time.  The time that is spent doing this is considerably longer than the length of the input pulse.  For typical operation of an MCA an average of approximately 10 s per event is required for analysis.  The multichannel analyzer is therefore “busy” after receiving a pulse and succeeding pulses during the “busy time” are lost.  For high counting rates this can be an appreciable portion of the time.  For example, with a random count rate of 104 per second, approximately 10% of the events would be lost.  Analyzers have clocks which can be preset to certain “live” times rather than actual times so that the true counting rates can be determined.

The multichannel analyzer (MCA) used for this experiment is a PC plug-in board coupled with software provided by Oxford Instruments.  This device is also used for Experiments VIII and IX. A brief description of the use of the MCA can be found in Appendix C of the lab manual. For a more detailed description of the MCA program and its consult the manual or the on-line help files. Because of the sophistication of the MCA, we suggest that you start the experiment when one of the instructors in present.

### The Pulse-Height Spectrum for 137Cs

137Cs is an isotope which undergoes decay with a half-life of 27 years.  It decays 92% of the time to the first excited state of its daughter nucleus 137Ba which, in turn, decays to the ground state by emitting a -ray of 0.662 MeV.  There is, therefore, one single -ray energy, which greatly simplifies the spectrum and its interpretation (see Fig. VII-8).

Figure VII-8

NOTE: To satisfy the Radiation Safety requirements, a Geiger counter must be operating in the vicinity of the experiments so that its audio signals will be a reminder that there is radiation in the area.  As the counter is battery powered, make sure both batteries are off when the counter is not being used.

Fix the standard source at a distance of roughly 10 cm from the face of the detector on the axis of the detector.  In determining the source-to-detector distance and the time interval to be used note the following:

- If the source is too close to the detector, the high counting rate increases the possibility of two or more pulses “piling up” and being counted as a single pulse with an amplitude greater than either individual pulse;

- If the source is too far from the detector, the counting rate due to the source will be comparable to the background rate.  The true source spectrum will not be easily determined, therefore;

- If the total number of counts recorded for a given time and channel is N, the percentage error in N will be 100 (N/N).  (Why?)  The time interval should thus be sufficient to make the average percentage error less than 5%.

Before starting to take data it is instructive to examine the anode pulses.  Set H.V. = +900 volts and use the oscilloscope to examine the time structure and pulse height of the traces.  Notice a bright band near the maximum.  This corresponds to the pulses in the photopeak.  The lesser pulses correspond to the Compton scattered events and to the backscatter peak.  Remove the source and notice the tube noise pulses.  Compare the pulse height with the approximate estimates given above.

Return the source to its previous place and examine the AMP output signals from the 490B.  Set the H.V. to a modest +700 volts and adjust the amplifier gain until the photopeak band is approximately 7 volts or a little bit less.  This will assure that the peak will occur toward the top of the MCA, but also assure that the amplifier will not get saturated. If you see a flat line at 7 volts, then most likely the amplifier is saturating.  If necessary, adjust the pole-zero compensation on the amplifier.

Measure the 137Cs pulse height spectrum with the MCA.  Save the spectrum on disk.  Take another measurement over the same time-period with the source removed.  Save this background spectrum on disk.

A useful measure of the relative number of -rays being detected via the photoelectric interaction is the peak-to-total ratio, P, defined as (the number of counts in the photopeak)/(the total number of counts in the spectrum).  Calculate this quantity for your measured spectrum, and compare to figure VII-9.   Note that the backscatter peak results from Compton scattering in the source and is therefore not part of the response of the NaI to a monochromatic gamma-ray.

You should calibrate the spectrum with sources of known -ray energies.  In order to calibrate the energy scale, you should obtain spectra for at least three different sources listed in Table 1.  Recognise the peaks in each spectrum as described in the table.  Draw a calibration curve with the corresponding channel number in the x-axis and the peak energy in terms of keV or MeV in the y-axis.  Correct for any zero error that might be present.  This would enable you to definitely identify the backscatter peak and other peaks not coming from the source.  Use the program ANALYSIS  or the Mathematica notebook provided for the course to subtract the spectrum taken without the source from the one taken with the source in order to remove the background contribution.

In the Cs spectrum, all the peaks should be correctly recognized.  It is not difficult to find the photo-peak.  Occasionally, students find extra peaks appearing on the left-hand-side of the spectrum, which leads to wrong recognition of peaks.  The backscattered peak can be easily found by using the expression for E´ in Eq. VII-4.  Recognize the Compton peak.  Any extra peaks might be due to the computer program, scattering from the lead shielding, or from the impurities in the source.  Bug the professor about it.

The peak-to-total ratio will clearly be a function of energy, since the photoelectric, Compton, and pair-production interaction processes depend on energy.  It will also be a function of the size and situation of the detector, since gammas and electrons lost through the sides of the detector and/or scattered into the detector from the surroundings will not produce counts in the photopeak.  Thus, the peak-to-total ratio will be, in a certain sense, a measurement of the quality and efficiency of detection versus energy.  Compare your measured value of P with a value for a similar size NaI crystal as shown in Fig. VII-9.

Figure VII-9

### Measurement of the Half-Life of 116In

The isotope about to be studied is a neutron irradiated sample of 115In which has been encapsulated in epoxy.  It is irradiated in the Graduate Laboratory by being placed near a Pu-Be source for a minimum time of one hour.  This is done prior to the laboratory period by a technician or the lab coordinator, and must be arranged in advance.  The sample will be brought to the experiment on request.  It should be left in the cabinet on completion of the experiment.

The 115In readily accepts a neutron and becomes 116In, which decays with a 54-minute half-life by the emission of an electron, to become an excited state of 116Sn.  In this experiment the -rays emitted in the decay of these excited states will be observed.  Since the number of gammas emitted is directly proportional to the number of transmutations, this allows the rate at which the 116In is decaying to 116Sn to be measured.

If the number of 116In nuclei at any time, t, is N(t), the number of transmutations in the next interval of time, dt, is

dN=−λN(t)dt			VII-5

where  is a constant.  Thus

N(t)=N0e−λt				VII-6

and N0 is the original number of 116In nuclei at a time zero.  The half-life of the isotope is defined as the time required for half the original nuclei to decay, thus

τ12=0.693λ .				VII-7

The MCA can also be utilized as a multichannel scaler (MCS).  In this mode the MCA scales the counts from a single channel analyzer for a fixed period of time.  The total number of counts in this period is stored in one channel.  The channel number is then advanced and the counts for the next time interval stored.  In this fashion one obtains the counts per fixed time interval as a function of time (i.e., channel number).  This is an ideal method for measuring the decay time of the radioactive source.

The object of this section of the lab is to measure the half-life of 116In.  The following is a procedure for making this measurement.  The prudent student may wish to take a short run in the multichannel analyzer mode to examine the -ray spectrum before proceeding to the half-life measurement. You may also want to practice using the MCS mode with another source before requesting the Indium source, since your best measurements with Indium will occur soon after it has been removed from the Pu-Be source.

Place the source approximately 10 cm from the NaI crystal.  Set the gain of the amplifier so that the pulses do not exceed about 7 volts.  Set the SCA DIFF/INT switch to INT with a baseline of about 0.5 volts.  With a BNC cable connect the output of the SCA to the MCS Data Input at the rear panel auxiliary connector of the MCA (this may already be done for you).

- Put the MCA in the multichannel scaling mode.

- Take the source away and examine background levels.  If the background is less than 5% of your source rate, forget it.  If it is higher, then you must measure the background in a second pass of the MCS.

- See Appendix C (or the additional lab handout) for more information on operation of the MCA in MCS mode.

- Write the complete set of data to disk for subsequent analysis.

Once the data have been acquired and transferred, to obtain the half-life, plot the gamma activity (i.e., number of counts per unit time) as a function of time on a semilog plot, and perform a linear fit to the data.  Alternatively, the half-life and its error can be obtained from the original data by fitting the data to an exponential form using a fitting program.

| Standard Sources |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Isotope | Energy of  emitted | Strength (Ci) | Number of ’s emitted per sec. | Date | Half-life |  |
| 57Co | 122 keV + 7%, 136 keV | 7.9 | 2.32  105  10% | 5/10/94 | 270 days |  |
| 137Cs | 662 keV | 7.1 | 2.63  105  10% | 5/5/94 | 28.8 yrs. |  |
| 54Mn | 835 keV | 7.9 | 2.84  105  10% | 7/28/94 | 314 days |  |
| 22Na | 511 keV | 7.6 | 1.94  105  10% | 11/3/93 | 2.60 yrs. |  |
|  | 1275 keV |  | 9.74  104  10% |  |  |  |
| 60Co | 1173 keV | 0.85 | 1.57  104  10% | 7/20/94 | 5.26 yrs |  |
|  | 1332 keV |  | 1.57  104  10% |  |  |  |

Table 1