# Introduction to Nuclear Physics and Gamma-Ray Spectroscopy: Part II

## References

Gamma-Ray Spectrum Catalogue: R. L. Heath (laboratory copy)

Nuclear Spectroscopy, Part A, edited by Fay Ajzenberg-Selove, pp. 32-38.

Nuclear Spectroscopy, Part B, pp. 211-227.

Alpha-, Beta-, Gamma-Ray Spectroscopy, edited by Kai Siegbahn, Vol. 1, pp. 245-302 (also in an earlier edition called Beta- and Gamma-Ray Spectroscopy, on pp. 132-182.

The History of Man-Made Radiation, by T. E. Allibone. Bulletin IP+PS 18, 1967.  Laboratory copy.

Also see footnotes #3 and 4 of this Experiment.

General modern physics texts or introductory nuclear physics texts that might be of general interest in understanding the experiment are available by many authors, e.g. Eisberg, Enge, Evans, Halliday and many others.

NOTE:  Before attempting this experiment you should perform experiment VII. This will provide the necessary introduction to the experimental techniques of gamma-ray detection.

## Preparatory Questions

(must be answered in lab book before experiment is started and signed by instructor or TA)

- Complete the problem described below.

- Explain briefly how one calibrates the energy scale of the multi-channel analyzer.

## Introduction

Experiment VIII is designed to further acquaint you with the use of a multichannel analyzer, to examine the detector response to gamma rays of varying energies, to study the efficiency of a NaI scintillator and finally to utilize these techniques in the study of gamma rays from a complex source.  The NaI(Tl) detector will obviously not detect all gamma-rays emitted from the source.  Only those emitted within the solid angle cone subtended by the face of the detector have any possibility of being detected (see Fig. VIII-1). Even these have some probability of passing through the detector without any inter-action taking place.  A useful quantity is the total detection efficiency, T, which is defined as the fraction of gamma-rays emitted from the source which interact with the detector.  This will not only depend on the source-detector geometry, but also the probability that the -ray will lose some of its energy once it enters the detector.

Figure VIII-1

## Detector Efficiency

Problem (to be done before the lab period).

Calculate the total efficiency of the detector.  This will involve determining the probability that a given gamma ray impinges on the detector times the probability that it then loses some of its energy in the NaI crystal.

Let N(x) be the number of photons per cm2 at x travelling along the x axis.  The number dN of photons which interact, and hence lose energy in a distance dx between x and x+ dx is proportional to N(x) and the distance dx.

dN(x)=−μN(x)dx					VIII-1

The factor of proportionality () is called the total linear attenuation coefficient.  Solving the differential equation subject to the boundary condition that N(x) = N0 at x = 0 we get

N(x)=N0e−μx.					VIII-2

The fraction of photons which interact in traversing a distance x' is then

N0−N(x')N0=1−e−μx'				VIII-3

The total linear attenuation coefficient includes contributions from pair production, photoelectric and Compton interactions.  The linear attenuation coefficients for sodium iodide are shown in Fig. VIII-2.

Figure VIII-2

If the detector thickness is x´ for all incident gamma-rays the total efficiency is given by (/4)(1ex’) where  is the solid angle at the source subtended by the face of the NaI detector.  Actually the NaI crystal is a right cylinder and presents varying thicknesses x depending on the angle  (see Fig. VIII-1).  For large distances (h0) relative to the dimensions of the detector one can obtain the approximate efficiency by assuming that the thickness is constant and equal to d.  The sodium iodide crystal used in this experiment is 3 inches long.

Calculate its efficiency as a function of the energy of the incident gamma-ray (0.1 MeV to 10 MeV) and bring a graph of the points with you to the laboratory class.

The photopeak efficiency can be defined as

εP = gammas in the photopeakgammas from the source = εT P			VIII-4

Calculate a value for this quantity using your calculated value for T at the 137Cs gamma-ray energy of 0.662 MeV and the peak-to-total ratio P determined in Experiment VII.

The emission rate of a source can be experimentally determined using the expression

N0=NPεP					VIII-5

Figure VIII-3

where NP is the number of counts in the photopeak.  For the 137Cs source calculate this quantity and compare it with the stated source activity.  Explain any discrepancies.  What are the advantages (if any) of using NP and P rather than the total number of counts and T?  (Suppose the source was not monoenergetic.)  Compare your value of T for the 0.662 MeV gamma-ray energy and your source-to-detector distance with that shown in Fig. VIII-3.  Explain the differences.

## Response of NaI Crystal as a Function of Energy

Using the Multichannel analyzer (MCA) (see the method discussed in Experiment VII), record the spectrum of 22Na, making sure that the gain setting permits observation of both peaks.  Save the spectrum on disk.  Obtain the location, FWHM, and number of counts for the two peaks.  Without changing the gain, record and save successively the spectra of 60Co and 137Cs obtaining the same information.  Using the 22Na spectrum as a calibration, determine the energies of the peaks in the 60Co and 137Cs spectra.  Determine the zero pulse height channel of the analyzer.  The 22Na spectrum results from gamma rays of energy 0.511 MeV and 1.275 MeV.

Find the peak-to-total ratio, photopeak efficiency, and activity for each of the gamma-ray energies.  Note how the first two quantities vary with energy.  To obtain the peak-to-total ratio for a source with two -rays, you must attempt to extract the separate Compton spectrum for each of the -rays (see Fig. VIII-5).  Be sure to correct for source decay, when necessary, in calculating the activities.  Plot the percentage energy resolution (FWHM/peak location corrected for zero channel) as a function of -ray energy.

Questions (Must be answered in writeup)

- Regarding the relationship of the measured and calculated efficiencies, do you think your measured efficiencies are reasonable in light of the approximation you made in the calculated efficiencies using (1ex´)?  How could you improve the calculation of ?  In what energy region is the approximation most valid?

- What is the dominant uncertainty in the measured efficiency?

- Why would it be difficult to calculate the photopeak efficiency?

- Based on the description of NaI detectors in Experiment VII, how would you expect the percentage energy resolution to depend on energy?

## Study of the Decay of 56Mn

In Experiment VII, you have studied the detection of gamma rays with a NaI(Tl) detector and the general statistical nature of radio-active decay.

In this section we will use these techniques to study some of the basic properties of nuclei and nuclear transitions.  A 55Mn sample will be placed near the neutron source; some of the 55Mn nuclei will capture a neutron and form the radioactive 56Mn (Half-life = 2.58 hrs.).  The 56Mn atoms are unstable and will decay by - emission to various states of 56Fe.  The excited states (energy levels of 56Fe) will decay by  emission either directly to the ground state or cascade by successive -decays through one or more other excited states to the ground state.

A decay scheme for 56Mn showing the major transitions is presented in Fig. VIII-4.  Note that the parent nucleus can decay to many different states in the daughter nucleus.  (In fact, it can decay to any state so long as conservation of energy, including rest mass, is not violated.)

In general the parent nucleus (56Mn) will -decay to those states in the product nucleus (56Fe) that are similar in character to it (e.g. small changes in angular momentum are preferred.)  Also, the larger the energy change the more favored the transition.  Similar selection rules and restrictions apply to the gamma decay within the product nucleus (56Fe).  Thus, by observing the relative intensities of the various and transitions that actually occur and noting those transitions which are very weak or absent (although energetically allowed), information concerning the similarity of different states be inferred.  More complex investigations of these transitions (e.g. coincidence or angular correlation studies) can be used to derive additional information.

In this experiment we will study only the energies and relative intensities of the -rays emitted.  (The  particles will not penetrate into the detector.)  By studying the energies and relative intensities of the various gamma rays, it is possible to determine the various energy levels involved in the gamma decay.  It is further possible to deduce the relative probability that each of these levels is excited in the beta decay process, by calculating the difference in the number of -rays feeding and decaying from a particular level.  (In the case studied here you will thus find all of the  decays, since there is no appreciable direct  decay to the ground state of 56Fe, which would not result in any -ray transitions.  The absence of this transition is due to the large angular momentum change, 4 units, required for it.)

### Preparation of the Mn Source

This source is prepared in the same way as the indium source; however, since it has a longer half-life, a bombardment of up to 3 hours is suggested.  Therefore if you wish to use this sample at the beginning of a lab period, you should arrange to have it placed in the neutron source ahead of time.  Like the indium, it should be left in the cabinet with the other sources at the end of the laboratory period.

### Experimental Setup

The setup is the same as was previously used with the multichannel analyzer.

To study all the gamma rays emitted from 56Mn it is necessary that the amplifier gain setting (and/or high voltage) be adjusted so that the energy range of the multichannel analyzer extends to about 3.5 MeV.  This can be done at the time the energy scale of the analyzer is calibrated.  In order to accurately determine the -ray energies, it will be necessary to calibrate the multichannel analyzer with the standard

Figure VIII-4

energy sources; thus it is important to maintain approximately the same counting rate for all sources to avoid gain changes due to different counting rates.  Use the 228Th source for calibration.  Peaks from that source ranging from about 240 keV on up to 2.6 MeV should be visible, with the latter lying somewhere around channel 360.   Note that the 228Th source available in the lab is rather weak, but if you put the source directly against the face of the NaI detector you should be able to get enough counts to find the 2.6 MeV peak in a reasonable length of time (10 minutes). Check the accuracy of the calibration by recording a 60Co spectrum at the same gain setting and determining the energy of the 60Co gamma rays.  This preliminary calibration and gain check should be done (i.e. a rough graph made in the lab) before proceeding any further.  Both a long (5-10 min) run and a shorter run must be taken in order to measure the weak lines and the strong lower energy line.  You should save all runs on disk.  In the long run the strongest line may overflow the memory several times, so the shorter run is required to normalize the intensity of this line to the others.  When calculating the true intensities of the measured transitions the total absorption efficiency vs. energy curves in Fig. VIII-3 should be used in conjunction with the peak-to-total curves in Fig. VII-9.

With the energy measurements of the transitions (including your estimate of the error) it should be possible to construct a decay scheme composed of cascade (two-step process) and cross over (one-step process) between the same initial and final states in 56Fe.  In deciding the order of the states, the intensities of the  transitions will help to assign the lower state.  Working backwards from the excess of  rays out of a state compared to those entering, you should be able to also construct the various  decay branches and their intensities from the ground state of 56Mn.  (Note again that there is no direct feeding of the ground state of 56Fe.)  These results should be compared with the results of C. Chasman and R. Ristinen where a higher resolution detector was used to study this decay scheme.  (Only that part of the paper concerned with 56Mn, not that concerned with 56Co, should correspond to your study.)  Another source of information of this decay scheme is Nuclear Data Sheets (A=56).

What inconsistencies, if any, do you note between your results and those given in the references?  How might these be reconciled?

As a final part of this section of the experiment describe or speculate upon what further experiment could be made on this source to either confirm or give more information on the decay scheme you have derived.  Estimate also any changes in technique or equipment that could have been used to improve the results of your experiment.  Explain why!

### Multiple-Energy Gamma-Ray Sources

If a NaI detector is exposed to a source emitting two different energy gamma rays, the resulting spectrum will be just the superposition of the spectra that would result from separate monoenergetic sources of the two energies.

For example, consider that the response of the detector to gamma rays of energy E1 is shown in Fig. VIII-5a while the response to gamma rays of energy E2 is shown in Fig. VIII-5b.  When a mixed source of both energies, E1 and E2, is studied, the spectrum shown in Fig. VIII-5c might result.  This is just the superposition of (a) and (b) and could be resolved as shown in Fig. VIII-d.  Thus the number of counts in the peak NP1 could be used to determine the activity of gamma rays of energy E1.

N1=NP1εP1					VIII-6

For the 22Na and 60Co spectra there is a fairly large probability, equal to T1T2 (/4)2 , of detecting gamma rays of the two different energies at the same time.  This will cause a third peak to be found at a channel corresponding to the sum of the gamma ray energies.  The total number of counts associated with the third peak must be added to the totals for both of the “real” peaks to find the source activities.  Since the number of counts in the third peak is proportional to 2, it can be decreased by increasing the source-to-detector distance.

Figure VIII-5

The complete gamma spectrum associated with the decay of 228Th is given in University of Maryland Technical Report #817 by R. O. Head, a copy of which may be found in the laboratory.  Note that the spectra in that report were obtained using a much higher resolution type of detector than the one you will actually be using.

C. Chasman and R. Ristinen, The Physical Review, Vol. 159 (1966), p. 915.

Nuclear Data Sheets, edited by K. Way, A=56