1:  Identify each of these logic gates by name, and complete the tables

- For the following Boolean expressions:

  - Construct the truth table

  - Write the Verilog expression

  - Simplify if possible

  - Draw the equivalent circuit in gates

- For the following circuit:

  - Construct the truth table

  - Construct the Boolean expression, starting with the SoP, then simplifying

  - Write the Verilog expression

- For the following 3 truth tables:

  - Construct the SoP Boolean expression and simplify

  - Write the Verilog expression

  - Draw the circuit in gates

- For the following truth table, design the circuit with the fewest possible gates.

- For the following PoS expressions, simplify and draw the resultant circuits:

- The following truth table describes a NAND gate:

Write the SoP and PoS expressions separately and compare.  When you’ve completed that, you will see why sometimes PoS is better.  For the next 2 truth tables, do the same:  write the SoP and PoS expressions, and use Demorgan’s theorem to show that they are equivalent.  Simplify and draw the circuit in gates.

- A seven segment decoder is a digital circuit designed to drive a very common type of digital display device: a set of LED or LCD segments that render the numerals 0 through 9 at the command of a four-bit code:

  - Why a 4-bit code?  Would a 3-bit code do?

  - Write the truth table that describes the logic needed to turn the bit code into the corresponding display number, assuming that a “1” means the segment is “lit”

  - Write the Boolean equation for each of the 7 outputs using either SoP or PoS, your choice, and simplify.

  - Draw the corresponding circuits.